1. INTRODUCTION

2. SAFETY

3. PROCESS OVERVIEW

4. TECHNICAL INFORMATION

5. SYSTEM OPERATIONS

6. TROUBLE SHOOTING

7. PREVENTIVE MAINTENANCE SCHEDULES

8. MAINTENANCE AND ADJUSTMENT INFORMATON

9. PARTS – SPARE PARTS

10. WARRANTY

11. CHEMCUT CONTACTS

TECHNICAL

DOCUMENTATION

Cc2002 - 60 inch Stations

SERIAL # c60-500

Ferric Chloride Etch

Atlas Die, LLC.

| Revision Status: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: | 0.1 |  | Date printed: | 12-10-03 |

Chemcut Corp.

500 Science Park Road.

State College, Pa. 16803-2299

Introduction

# Contents..………………..……………………………………………………………….Page

1.1	Scope of the Manual	3

1.2	Navigation:	3

1.3	Symbols	3

1.4	Diagrams	4

1.5	Additional Documentation	4

1.5.1	Electrical	4

1.5.2	Chemicals	4

Scope of the Manual

This manual explains the operation, maintenance, and parts identification procedures for your Chemcut processing system.  This Manual is divided into several chapters.  The subject content of each chapter is listed in the “Table of Contents” on the cover page of this document (page 1).  Other important information contained on the cover page includes the system serial number and the intended processes for your Chemcut system.

Navigation:

Navigation through this CD based document, is accomplished by “pointing” and “clicking” at links contained within the document.  The central point of navigation is the “Title Page” (page 1-1).  Positioning the cursor over a chapter heading (pointing), and then depressing the left mouse button, (clicking) will automatically cause the document to jump to the start of the selected chapter.

Each chapter contains links to further refine the navigation through this document. “Pointing” and “Clicking” on any page number in a chapter index page will cause the document to jump to the requested page or section of the document.  Returning to the “Title Page”, is accomplished by selecting the first page of this document.  Clicking on the “first page” button located on the Adobe toolbar, or sliding the vertical scroll bar to its very top position are the best methods for returning to the “Title Page”.

Symbols

Two symbols, placed on the right side of the page are used in this documentation to highlight points that should receive special attention:

To avoid health hazards and safety risks for personnel,

please carefully observe comments

indicated with the following symbol:

Points that are particularly important to the

operation and maintenance of the equipment

are marked with the following symbol:

Diagrams

The standard transport direction is from left to right.  Left to right transport direction is defined as the direction of product flow as viewed from the operator’s side of the equipment.  Reference diagrams included in this documentation are illustrated based on this left to right transport direction.  For right to left machines, the concepts illustrated on these drawings are correct, but need to be visualized as mirror images.  To help in the comprehension of these drawings, it is important to consider the “Transport Direction” that is indicated on these diagrams.

Additional Documentation

Electrical

Printed copies of the documents below are located in the electrical control cabinets:

Layout diagram

Schematic wiring diagrams (main and control circuits)

Parts lists (electrical)

Note:  In addition to the printed copies provided in the electrical cabinets, digital copies of this documentation and viewing software is provided on this CD.

Chemicals

The documents below related to chemicals are supplied with the chemicals.  They are not part of this document:

Chemical safety sheets

Instructions for the use of chemicals

Analysis procedures

Safety

# Contents..…………………..………………………..………………………………..…Page

2.1	General information	6

2.1.1	Responsibility of the owner	6

2.1.2	Responsibility of operators and maintenance personnel	6

2.1.3	Design application	7

2.1.4	Guarantee and liability	7

2.2	Safety and protection devices	8

2.2.1	Emergency stop switches	8

2.3	Training of personnel	8

2.4	Equipment control	9

2.5	Safety procedures	9

2.5.1	General	9

2.5.2	Safety procedures in normal operation	10

2.5.3	Safety procedures for the electrical system	11

2.5.4	Safety procedures for chemicals 	11

2.5.5	Safety procedures for gases and vapors	12

2.5.6	Safety procedures for parts at elevated temperatures	12

2.5.7	Safety procedures for moving parts	13

2.5.8	Safety procedures for accessing internal parts	14

2.6	Maintenance and repair	16

2.7	Constructional changes to the equipment	16

2.8	Cleaning the equipment and material disposal	17

2.9	Emergency procedures	17

2.9.1	General	17

2.9.2	Panel Jam	17

2.9.3	Clothing or body parts caught in moving parts	18

2.9.4	Personnel in contact with process solution	18

General information

This documentation contains important information about how to operate and maintain Chemcut equipment safely.

In order to operate Chemcut equipment in a safe and trouble-free manner, a thorough knowledge of safety instructions and regulations is required.

All personnel who work on or operate Chemcut equipment should follow the instructions in this documentation, particularly in regard to safety.

In addition, please obey Federal, State, Local rules and regulations concerning accident prevention.

Responsibility of the owner

The owner is responsible for giving permission to operate and/or service Chemcut equipment only to those individuals who:

are fully qualified and trained to operate or service the equipment.

are fully aware of the regulations concerning safety at work and accident prevention.

have read and understood the safety chapter and the safety instructions in this operation manual.

The safety awareness of the personnel operating and working on the machine should be verified on a regular basis.

All safety features on the equipment must be checked on a regular basis.

The owner must provide all necessary personal protection equipment and training in the proper operation of this equipment.

Responsibility of operators and maintenance personnel

All personnel who operate or service the machine, are responsible for:

following all relevant regulations concerning safety at work and accident prevention.

reading the safety chapter and safety instructions in this operation manual.

Design application

This Chemcut equipment is solely designed for the processing of printed circuit boards through the process(es) indicated on the cover sheet, (page 1) of this document.  The use of the equipment for other purposes, or the use of the system beyond its stated specifications is not recommended, and is beyond the scope of the design intent of the equipment.

In any such case, Chemcut shall not be responsible for any damage and/or lost productivity caused by the misuse or misapplication of the equipment.

In addition, proper use of the equipment includes that:

all instructions in this documentation must be followed.

all necessary inspection and maintenance work has to be carried out in a proper manner and in compliance with the provided maintenance schedules.

Guarantee and liability

The CHEMCUT  "General Conditions of Sale and Delivery" are absolutely binding. These are available to the owner either before or at the time of signing the contract. Guarantee and liability claims for injury to personnel or damage to equipment cannot be accepted if these arise from one or more of the following causes:

the equipment is not used for its designed application

improper operation and/or maintenance of the equipment

operation of the equipment with defective safety devices or incorrectly mounted or non-functioning safety and protective devices

non-compliance with the instructions in this documentation in regards to start-up, operation, maintenance and set-up of the equipment

unapproved constructional changes made to the equipment by the owner

insufficient monitoring of equipment parts which are subject to wear- and tear

repair work carried out incompetently

damage caused by foreign bodies or excessive force being applied to the equipment, including natural causes.

For additional information, please refer to the “Warranty” section of this documentation.

Safety and protection devices

Before every equipment start-up it must be ensured that all safety and protection devices are properly mounted and in good working order.

Safety and protection devices may only be removed;

after the machine has stopped completely and

after ensuring that the machine cannot be restarted.

For components that are not supplied by CHEMCUT, the customer must install safety and/or protective devices according to the appropriate regulations.

Emergency stop switches

Emergency stop switches are mounted at several positions in the line. (Input and output ends, in the middle of the operator side and near the dosing units on the service side of the line etc).

Emergency stop switches have red activators and are mounted on a yellow background.

Emergency stop switches comply with German VDE 0100, VGB 4, and other international requirements.

The operation of any emergency stop switch turns off the control voltage to all line components. All contactors for pumps, motors etc. will be switched off

Training of personnel

Only personnel who are specifically trained and instructed how to operate and maintain the equipment correctly may work on it.

The responsibilities of personnel for machine operation, set-up, maintenance and repair must be defined clearly.

Trainees may only work on the equipment under the supervision of experienced personnel.

Equipment control

The software program must not be changed under any circumstances!

Software functions, except the entering of product parameters, are password protected. It is recommended that the password be changed at least every 3 months!

Only trained and authorized personnel may operate the controls.

Safety procedures

General

Chemcut equipment is built according to current technical standards and prevailing safety regulations.  However, it may be possible that its operation could present risks to the health and safety of operators or third parties. The risk of damaging the machine or other items can also arise.

Chemcut equipment may only be used;

for the application for which it is designed

when all safety related functions are in proper working order

Defects that could be dangerous to safety must be fixed immediately.

This documentation must always be accessible in the immediate vicinity of the equipment.

In addition to this documentation, the general and local regulations regarding accident prevention and environmental protection must be available and adhered to.

All safety, warning notices and lettering attached to the equipment must be maintained in legible condition.

Safety procedures in normal operation

Check the equipment visually before starting up the line.  Verify that no one could be exposed to danger by machine start-up.

The equipment may only be operated when all the safety and protection devices are fully able to function.

Never operate the equipment with any protective covers removed.

Never install a device or modify the electrical wiring for the purpose of over-riding a safety device.

At least once each shift, the operation of the Emergency Stop circuit should be verified and the equipment should be checked for any visible external damage.

All operators must know the locations of the Emergency Stop buttons, the main power disconnect switch, and any emergency shut off valves on the chemical solution lines on the system.

Always keep clothing and body parts away from all moving mechanical parts in the equipment, i.e. gears, shafts, etc.

Always shut off pumps prior to removing any of the spray chamber covers.

Wear eye protection at all times when working in the vicinity of the system.

Always wear the appropriate protective clothing when working with hazardous chemicals.

Never manually mix chemicals in the equipment sumps.  Some chemicals can release dangerous amounts of heat which could cause irreparable damage to the equipment.

Appropriate personnel must verify the proper operation of all interlocks, protection, and ventilation devices on the equipment on a regular basis.

Never operate the equipment when under the influence of drugs or alcohol.

Never operate the equipment at a temperature that is not compatible with the materials of construction.

Safety procedures for the electrical system

Only trained electricians may be permitted to work on the electrical system.

The equipment’s electrical system has to be checked regularly. Loose connections and damaged cables must be repaired immediately.

The control cabinet should always be kept locked. Access should only be permitted to authorized personnel.

Should work be necessary on live wires or equipment the local health and safety regulations must be observed.

Ensure that no tools or other metallic objects have been left on the equipment or in the electrical boxes.  These objects can be the source of short circuits.

Safety procedures for chemicals

Aggressive chemicals are used in Chemcut equipment. All instructions contained in the safety information sheets, operation manual, and analysis procedures must be followed.

Always wear the appropriate protective clothing when working with hazardous chemicals.

All solutions and chemicals that may be used in the equipment must be handled and disposed of in accordance with established safety procedures and regulations.

Maintenance work on modules may only be carried out after they have been emptied and cleaned.

Maintenance work on pipes may only be carried out when the pipes are not under pressure.

When taking samples for analysis purposes, protective clothing must be worn. The drive and pumps must be switched off.

Never manually mix chemicals in the equipment sumps.  Some chemicals can release dangerous amounts of heat  which could cause damage to the equipment.

Never operate the equipment with any chemistry or concentrations of chemistry that is not compatible with the materials of construction.

Safety procedures for gases and vapors

Should the exhaust system not be operating correctly, a release of dangerous gases and vapors can occur!

Proper personnel protective equipment for dealing with such a release should be available in the immediate area.

The performance of the exhaust system must be verified on a regular basis.

The verification of exhaust system performance must occur at each individual exhaust connection point along the processing line.

Safety procedures for parts at elevated temperatures

Some tanks or other pieces of equipment may operate at an elevated temperature.  Exposed surfaces pose a potential burn hazard.

Care should be taken to avoid contact with hot surfaces while working around an area operating at an elevated temperature.

Appropriate protective clothing must be worn when working on parts at elevated temperatures.

Pay attention to labels indicating a hot surface.

Safety procedures for moving parts

On all Chemcut equipment, the opening of certain lids, covers, and doors will immediately stop spray pumps.  This action dramatically reduces the danger of personnel being splashed or sprayed with chemicals.

On certain Chemcut equipment, the lifting of these covers will also stop the conveyor drive and oscillation mechanisms from moving.  These steps are designed to reduce the possibility of personnel getting clothing or body parts caught in moving mechanical parts.

The possibility of getting clothing or body parts entangled in moving parts is often referred to as a pinch hazard.  A pinch hazard may cause cuts, bruises, abrasions, and even the loss of appendages such as fingers.

CAUTION:  Extreme care should be taken to avoid contact with moving parts.

Safety procedures for accessing internal parts

This procedure MUST be followed when removing covers or doors from CHEMCUT equipment to access the internal processing area.

- Read chemistry manufacturer’s material safety data sheet.

- Follow chemistry manufacturer’s recommended handling procedures.

- Know your plant emergency procedures.

- Turn off conveyor on CHEMCUT equipment.

- Turn off spray pumps on CHEMCUT equipment.

- Wait 2 minutes to allow solution to drain from internal surfaces into sump.

- Put on personal protective equipment recommended by chemistry manufacturer which can include goggles, gloves, apron and respirator.

- Remove cover or door of CHEMCUT equipment.

- Replace cover or door when internal access is no longer required.

- Restart equipment in accordance with plant operating procedure.

Este procedimiento debe seguirse al quitar las tapas o las puertas delEquipo CHEMCUT para tener acceso a la superficie interna de procesamiento.

- Leer el MSDS (pliego de datos sobre la seguridad del material) Suministrado por el fabricante.

- Seguir procedimientos de manejo recomendados por el fabricante de laSubstancia quimica.

- Saber el procedimiento de emergencia de su planta.

- Apagar el transportador del equipo CHEMCUT.

- Apagar las bombas pulverizadoras del equipo CHEMCUT.

- Esperar 2 minutos para permitir el drenaje de la substancia quimica Desde la supericie interna  al sumidero.

- Colocarse el equipo de proteccion personal recomendado por el Fabricante de la substancia quimica, que puede incluir protrctores Oculares, guantes, delantal, y respirador.

- Quitar la tapa o la puerta del equipo CHEMCUT.

- Colocar nuevamente la tapa o la puerta cuando ya no sea necesario El acceso al interior.

- Poner en marcha nuevamente el equipo, de acuerdo con el Procedimiento de operacion de la planta.

Maintenance and repair

The required set-up, maintenance and inspection work must be carried out according to the maintenance schedules,              (see section 7, Preventative Maintenance Schedules)

Before conducting any maintenance, repair, or inspection work, the Chemcut line must be switched off and the main switch securely protected against unexpected activation:

switch off and lock the main switch

remove the key (if applicable)

inform the machine operator(s)

attach a warning sign to prevent switching on

When exchanging large and/or heavy items attach them carefully and securely to the lifting device.

Check loose screw connections for tight fit.

After finishing maintenance work, check all safety and protection devices for correct function.

Constructional changes to the equipment

No changes or modifications to the equipment may be made without prior permission of the manufacturer. This includes welding of supporting structural parts.

All modifications require written confirmation from Chemcut.

Immediately replace any part of the equipment that is not in proper working condition.

Only use original spare parts:

If parts are secured from suppliers other than Chemcut, Chemcut cannot guarantee that these parts have been designed and manufactured to meet the Chemcut’s performance and safety requirements.

Cleaning the equipment and material disposal

Only use cleaning agents that are compatible with the materials of construction.

All materials used must be handled and disposed of according to the relevant regulations and procedures.

Emergency procedures

General

An emergency is defined in these procedures as an immediate situation in which personal injury is occurring or eminent, or where equipment damage is occurring or eminent.

Panel Jam

IN THE EVENT OF A  PANEL JAM:

- Stop the system using the appropriate Stop button.

- Visually identify the area in which the jam has occurred.

- Put on the appropriate personnel protective clothing and/or device(s).

- Remove the necessary protective covers to gain access to the area where the jam has occurred.

- Remove the jammed material from the conveyor.

- After the jammed material has been removed from the equipment, inspect the conveyor rods in the area of the jam to be sure they are properly seated.

- Inspect the conveyor wheels to ensure they are correctly spaced on the rods.

- Replace the protective covers that were removed.

- Restart the system in accordance with the established procedures.

- Clean the equipment of any chemical splashes or spills that may have occurred while removing the jam.

Clothing or body parts caught in moving parts

IN THE EVENT OF CLOTHING OR BODY PARTS ARE CAUGHT IN MOVING

MECHANCAL PARTS:

- Stop the system using the appropriate Stop button.

- Render assistance to the person who is caught and immediately notify the appropriate emergency personnel in accordance with established emergency procedures.

- Disconnect all electrical power to the equipment as soon as possible after completing step 2.

Personnel in contact with process solution

IN THE EVENT OF PERSONNEL IN CONTACT WITH PROCESS SOLUTION:

(resulting from ruptured piping or some other abnormality)

- Stop the system using the appropriate Stop button.

- Render assistance to the person contacted by the solution and immediately notify the appropriate emergency personnel in accordance with established emergency procedures.

- Disconnect all electrical power to the equipment as soon as possible, after completing step 2.

- Lockout the main power disconnect switch in the off position until the proper repairs can be made.

Process Overview

# Contents..……………………………………………………..…………...……….… Page

3.1	General	20

3.2	Process steps	20

3.3	System design	20

3.4	PLE634  Load and Drive	20

3.5	PIA634  Conveyor Drive	20

3.6	POS607  Ferric Chloride Etch	21

3.7	POS607   Ferric Chloride Etch	22

3.8	PIA634  Conveyor Drive	23

3.9	PRE634  Unload	23

General

This Chemcut processing system provides for continuous, integrated; horizontal processing materials.  Chemcut’s global product offering designed specifically for highly reliable, continuous, mass production of High Density Interconnect products.

Process steps

The process sequence consists of following steps.

| Station. No. | Module Name | Process Step |
| --- | --- | --- |
| 1 | PLE634 | Load and Drive |
| 2 | PIA634 | Load and Drive |
| 3 | POS607 | Ferric Chloride Etch |
| 4 | POS607 | Ferric Chloride Etch |
| 5 | PIA634 | Conveyor Drive Unload |
| 6 | PRE634 | Unload |

System design

PLE634		Load and Drive

A. OVERALL LENGTH - 69.9"

B. 1" DRAINS PIPED TO STATION 2

C. EMERGENCY STOP BUTTON

D. 1/4 HP DC GEAR DRIVEN CONV. DRIVE MOTOR (ON THE PLE634)

E. DIGITAL DISPLAY OF CONV. SPEED

F. LINEAR SPEEDS TO 140 IPM

G. TITANIUM CONVEYOR MOTOR MOUNTING PLATE AND ADJUSTING BOLTS

PIA634		Conveyor Drive

A. OVERALL LENGTH - 69.9"

B. 1" DRAINS PIPED TO STATION 2

C. EMERGENCY STOP BUTTON

D. 1/4 HP DC GEAR DRIVEN CONV. DRIVE MOTOR (ON THE PLE634)

E. DIGITAL DISPLAY OF CONV. SPEED

F. LINEAR SPEEDS TO 140 IPM

G. TITANIUM CONVEYOR MOTOR MOUNTING PLATE AND ADJUSTING BOLTS

POS607		Ferric Chloride Etch

A. OVERALL LENGTH - 54.9"

B. SUMP CAPACITY - 215 GALLONS

C. CLEAR REMOVABLE FRONT DOOR WITH INNER SPLASH SHIELD

D. CLEAR REMOVABLE REAR ACCESS DOOR

E. REMOVABLE SEE-THROUGH TOP LIDS WITH EXIT

F. REED SWITCH INTERLOCKS ON ALL REMOVE ABLE LIDS AND DOORS

G. TWO UPPER AND LOWER SPRAY RACKS (ALL RACKS ARE REMOVEABLE

THROUGH THE  FRONT DOOR)

H. 70, 1.5 GPM CONE NOZZLES ON THE TOP  AND THE BOTTOM

I. FLOW RATE IS APPROX. 210 GPM @ 40 PSI

J. IND. SPRAY PRESSURE ADJ. FOR UPPER  INPUT,UPPER OUTPUT,LOWER

INPUT,AND  LOWER OUTPUT RACKS

K. FOUR FRONT READABLE PRESSURE GAUGES

L. ULTRAFINE SPRAY PRESSURE CONTROL INTERNAL INDIVIDUAL SPRAY

PRESSURE ADJ. FOR EACH SPRAY CHANNEL IN EACH  RACK (20 TOTAL).

TWENTY FRONT READABLE PRESSURE GAUGES, ONE FOR EACH  CHANNEL

M. TWO 7.5 HP CENTRIFUGAL FLUID PUMPS  WITH TITANIUM IMPELLERS

N. EXTERNAL FILTER UNITS W/100 MICRON BAG ELEMENTS BETWEEN SPRAY

PUMPS AND  SPRAY RACKS

O. RACKS OSCILLATE IN A PLANE PARALLEL TO THE WORK PIECE AND

PERPENDICULAR TO THE WORK FLOW

P. TWO SYNCHRONIZED, DIRECT GEAR DRIVEN OSCILLATION DRIVE MOTORS

Q. INDEPENDENT OSC. SPEED CONTROL (TO 54 SWEEPS/MINUTE)

R. DIGITAL TEMPERATURE CONTROLLER

S. 36 KW TOP MOUNTED TITANIUM HEATERS  FOR HEAT-UP FROM 60 DEG.F TO 125

DEG

F. IN 1 HOUR

T. OUTPUT DRAGOUT ROLLERS ARE DELETED

U. FOUR 6" SUMP INTERCONNECTS AT EXIT

V. 3 HP RECIRC. PUMP PIPED TO THE POS606  OF STATION 03

X. IX MODEL ETCH 1 SPECIFIC GRAVITY CONTROLLER (FED FROM RECIRCULATION

LOOP)  TO ACTIVATE WATER ADD TO ETCH CHAMBER  #1 TO MAINTAIN

SELECTED SPECIFIC GRA VITY

POS607		 Ferric Chloride Etch

A. OVERALL LENGTH - 54.9"

B. SUMP CAPACITY - 215 GALLONS

C. CLEAR REMOVABLE FRONT DOOR WITH INNER SPLASH SHIELD

D. CLEAR REMOVABLE REAR ACCESS DOOR

E. REMOVABLE SEE-THROUGH TOP LIDS WITH INNER SPLASH SHIELD AT

ENTRANCE AND EXIT

F. REED SWITCH INTERLOCKS ON ALL REMOVEABLE LIDS AND DOORS

G. TWO UPPER AND LOWER SPRAY RACKS (ALL RACKS ARE

REMOVEABLE THROUGH THE  FRONT DOOR)

H. 70, 1.5 GPM CONE NOZZLES ON THE TOP AND THE BOTTOM

I. FLOW RATE IS APPROX. 210 GPM @ 40 PSI

J. IND. SPRAY PRESSURE ADJ. FOR UPPER  INPUT,UPPER OUTPUT,LOWER\

INPUT,AND  LOWER OUTPUT RACKS

K. FOUR FRONT READABLE PRESSURE GAUGES

L. ULTRAFINE SPRAY PRESSURE CONTROL INTERNAL INDIVIDUAL SPRAY

PRESSURE  ADJ. FOR EACH SPRAY CHANNEL IN EACH RACK (20

TOTAL). TWENTY FRONT READABLE PRESSURE GAUGES, ONE FOR

EACH  CHANNEL

M. TWO 7.5 HP CENTRIFUGAL FLUID PUMPS WITH TITANIUM IMPELLERS

N. EXTERNAL FILTER UNITS W/100 MICRON BAG ELEMENTS BETWEEN

SPRAY PUMPS AND  SPRAY RACKS

O. RACKS OSCILLATE IN A PLANE PARALLEL TO THE WORK FLOW

P. TWO SYNCHRONIZED, DIRECT GEAR DRIVEN  OSCILLATION DRIVE

MOTORS

Q. INDEPENDENT OSC. SPEED CONTROL (TO 54

SWEEPS/MINUTE)

R. 36 KW TOP MOUNTED TITANIUM HEATERS FOR HEAT-UP FROM 60 DEG.F

TO 125 DEG

F. IN 1 HOUR

S. INPUT DRAGOUT ROLLERS ARE DELETED

T. FOUR 6" SUMP INTERCONNECTS AT ENTRANCE

PIA634		Conveyor Drive

- OVERALL LENGTH - 69.9"

B. 1" DRAINS PIPED TO STATION 3

C. EMERGENCY STOP BUTTON

D. 1/4 HP DC GEAR DRIVEN CONV. DRIVE  MOTOR (ON THE PRE634)

E. LINEAR SPEEDS TO 140 IPM

F. TITANIUM CONVEYOR MOTOR MOUNTING  PLATE AND ADJ. BOLTS

G. SLAVED TO THE DRIVE OF STATION 01

PRE634		Unload

- OVERALL LENGTH - 69.9"

B. 1" DRAINS PIPED TO STATION 3

C. EMERGENCY STOP BUTTON

D. 1/4 HP DC GEAR DRIVEN CONV. DRIVE  MOTOR (ON THE PRE634)

E. LINEAR SPEEDS TO 140 IPM

F. TITANIUM CONVEYOR MOTOR MOUNTING  PLATE AND ADJ. BOLTS

G. SLAVED TO THE DRIVE OF STATION 01

TECHNICAL INFORMATION

# Contents…………………………………….…………………………………………...Page

4.1	Utility Requirements	25

4.1.1	Overall Length	25

4.1.2	Overall Width	25

4.1.3	Conveyor Type	25

4.1.4	Conveyor Height	25

4.1.5	Conveyor Width	25

4.1.6	Electrical Power Supply	25

4.1.7	Estimated Power Requirements	25

4.1.8	Intermittent Cooling	26

4.1.9	Exhaust Venting	26

4.1.10	Conveyor Speed Range	26

4.1.11	Maximum Operating Temperature	26

4.1.12	Conveyor Direction	26

4.1.13	Product Size	26

4.1.14	Product Thickness	26

4.2	Layout Drawing	27

Utility Requirements

Operation of this processing system requires the following utilities.  It is the responsibility of the customer to provide these utilities.

Overall Length

249.67 in.

Overall Width

101.00 in.

Conveyor Type

Horizontal “Modified”

Conveyor Height

37.5 - 39 in.

Conveyor Width

Nominal	62 in.

Effective	60 in.

Electrical Power Supply

3 Phase 60 Hertz

480 volts

60 Hertz

Estimated Power Requirements

100 KVA

Intermittent Cooling

20 gpm @ 20 psi/50f. maximum

Exhaust Venting

600 cfm

Conveyor Speed Range

0-140 inches per minute

Maximum Operating Temperature

130º F in all stations

Conveyor Direction

Left to Right Operation

Product Size

Minimum	4” wide x 15”

Maximum	60” wide x any length

Product Thickness

0.015” core to 0.250”

Layout Drawing

System Operation

# Contents…………………………………………………………………………….Page

GENERAL	29

Manual / Auto Key Switch	29

Start / Stop Pushbuttons	30

Emergency Stop	30

Overview Screen General	30

Alarms	31

Navigation buttons	31

Pump / Motor Control	31

Alarm Acknowledge / Alarm Reset	32

Conveyor Runout:	32

Navigation buttons	32

Speed Control	33

Material Control	33

Heat & Cool	34

Fluid Level	34

Solenoid valves	34

Alarm Acknowledge / Alarm Reset	35

Alarm Ban Setpoint	35

Navigation buttons	35

Numerical Display	36

Temperature Control	37

Fluid Level Control	37

PH Control	37

SERVICE MODE	38

- GENERAL

This chapter covers the use and operation of the electrical controls package supplied with your Chemcut processing system.  This processing system utilizes a PLC based control package with a Panelview Operator Interface Panel (OIP).   The OIP is a touch screen graphical interface with custom designed screens for efficient and user-friendly operation.

The OIP is typically mounted in the door of the electrical enclosure, or optionally mounted on the input or output of the processing system.   In addition to the OIP, a MANUAL/AUTO key switch, START and STOP pushbuttons, and EMERGENCY STOP

controls are provided.  The operations of these controls are described below.

- Manual / Auto Key Switch

MANUAL

AUTO

The MANUAL/AUTO key switch selects the operating mode.  In the MANUAL mode, individual pumps and heaters may be configured for either an ON of OFF state and parameters such as temperature and conveyor speed are adjustable.   In the AUTO mode configuring of pumps and heaters and the adjustment of process parameters is restricted.

- Start / Stop Pushbuttons

STOP

START

The START and STOP pushbuttons are used to start and stop the line,  After the line is configured using the OIP, pressing START will apply power to all the configured devices provided that no interlocks (alarms) are present.  The STOP button removes power from configured devices.

A STOP condition is automatically triggered in the event of an interlock

(alarm).

- Emergency Stop

The EMERGENCY STOP pushbutton is to be used should an event require that all machine motion must stop immediately and/or it is necessary to remove hazardous voltage from the machine.   EMERGENCY STOP should not be used as a normal machine shutdown as it may interfere with time out sequences designed to remove excess heat from the machine prior to shut down.  The STOP button should be used for normal machine shutdown.

- Overview Screen General

The SYSTEM OVERVIEW screen is the HOME or MAIN screen for the system.  This screen provides some overall information on the status of the processing system and serves as the central navigation point for accessing information from all the processing stations.

Navigation from this screen to all the processing stations is accomplish by touching the

graphic representation of the processing station.  Other navigation buttons and status indicators are explained below:

Normal operational status of a processing station is indicated with yellow text on a gray background as shown by the ETCH CLEANER graphic on the left above.  A processing station that is in a  WARNING or ALARM state will be indicated with red text on a yellow background as illustrated by the ETCH CLEANER graphic on the right above.

- Alarms

NORMAL operational status for the line is indicated by green status lights as shown above on the left side.  FAULT conditions are indicated by red status lights as illustrated by the EMERGENCY STOP light shown on the right side above.

- Navigation buttons

Pressing this button will select the ALARM screen.  For further information, refer to the ALARM screen section in this chapter.

Pressing this button will select the PROCESS DATA screen.  For further information, refer to the PROCESS DATA screen section in this chapter.

Pressing this button will select the RECIPIES screen.  For further information, refer to the RECIPES screen section in this chapter.

- Pump / Motor Control

OFF			             ON

Pump / Motor controls are visualized with two states.  In the “OFF” state the graphic is all gray.   Pressing on the graphic, causes the control to switch to the “ENABLED” state that is indicated by the graphic turning yellow.  Pressing the graphic again causes the graphic to return to the “OFF” state.

- Alarm Acknowledge / Alarm Reset

Pressing this button silences any audible alarm and/or light beacon that was triggered by an Alarm or Warning condition.

Following the repair of the issue that caused the Alarm or Warning condition, pressing this button will clear the Alarm or Warning condition and allow normal machine operation to be re-started.

- Conveyor Runout:

Pressing this button during most ALARM conditions will start the conveyor to permit the operator to exit product from the line.  This button only functions during alarm conditions.  This button will not function with a conveyor motor overload or Emergency Stop condition present on the line.

- Navigation buttons

Pressing the 	    steps back one processing station.  Pressing the

advances to the next processing station’s screen.

Pressing this button will select the ALARM screen.  For further information, refer to the ALARM screen section in this chapter.

Pressing this button will select the SPEED CONTROL screen.  For further information, refer to the SPEED CONTROL screen section below.

Pressing this button will select the MATERIAL CONTROL screen.  For further information, refer to the MATERIAL CONTROL screen below.

Pressing the “CLOSE” button will close the current screen and return to the previous screen or to the SYSTEM OVERVIEW screen.

- Speed Control

This screen permits the adjustment of the Conveyor Speed Setpoint and the High and Low Warning windows.   The SETPOINT field is used to set the conveyor speed.  The setpoint is entered by keying in the desired speed in engineering units

- Material Control

This screen permits the adjustment of the Total Count and Jam Process.

Pressing any of the three fields on the SPEED CONTROL screen, (SETPOINT:, WARNING HIGH: or WARNING LOW:) causes the following numeric entry screen to appear:

BACK KEY							 RETURN KEY

IMPORTANT NOTE!

A default value of 0.0 is displayed in the top window every time the numeric entry screen is accessed.    Pressing the RETURN key prior to entering a new value, will send the value 0.0 to the PLC.  The PLC will then default to the minimum value for the parameter. It is important to always enter the desired value prior to pressing the RETURN key. To exit the numeric entry screen without changing current values, use the BACK key prior to pressing any other keys.

To change the parameter value, simply key in the desired value using the numeric entry keypad.  For example, to enter a SETPOINT of 120.0 IPM, key in 120.0 followed by the ENTER key.  Values to be entered, must be between the minimum and maximum values that are displayed directly below the top window.  Values outside of this range will result in a “value out of range” message.

- Heat & Cool

OFF		    ENABLED		   HEATING		COOLING

The OFF state for HEATING and COOLING is displayed with a gray “OFF” box.  Pressing on the graphic causes the “OFF” box to turn to a yellow “ON” indicating an “ENABLED” state.  The actual “HEATING” of the bath will occur when the actual value of the bath temperature is less than the setpoint.  The “HEATING” state is indicated with a red box over the graphic representation of the heating element.  “COOLING” of the bath will occur when the bath temperature is greater than the setpoint.  “COOLING” is indicated with a cyan box over the graphic symbol representing the cooling coil.

- Fluid Level

The depth of the fluid in a tank in indicated both numerically and with a bar graph.   A digital number representing the depth of the solution is displayed in yellow text on a dark gray background.  A blue bar graph represents the percentage the bath is full.  A solid blue bar indicates a full bath while a thin bar of blue indicates a bath that is nearly empty.

- Solenoid valves

OFF						ON

“OFF” states of solenoid valves are indicated with a gray “OFF” box.  When the solenoid is energized, a green “ON” box appears.  Some solenoids are illustrated for monitoring purposes only, and cannot be turned ON or OFF from the graphic screen.  Nothing will happen when these graphics are pressed.    Other solenoids do have the ability to be manually turned ON and OFF from the graphic screen In the “Manual” mode of operation.   When these solenoid graphics are pressed in the “MANUAL” operating mode, the graphic will toggle to the “ON” state.  Pressing the graphic again will cause the solenoid to return to the “OFF” state.

- Alarm Acknowledge / Alarm Reset

Pressing this button silences any audible alarm and/or light beacon that was triggered by an Alarm or Warning condition.

Following the repair of the issue that caused the Alarm or Warning condition, pressing this button will clear the Alarm or Warning condition and allow normal machine operation to be re-started.

- Alarm Ban Setpoint

Pressing this button allows the adjustment of the high warning.

Pressing this button allows the setting of the actual setpoint.

Pressing this button allows the adjustment of the low warning.

- Navigation buttons

Pressing the 	    steps back one processing station.  Pressing the

advances to the next processing station’s screen.

Pressing this button will select the ALARM screen.  For further information, refer to the ALARM screen section in this chapter.

Pressing this button will select the TEMPERATURE CONTROL screen.  For further information, refer to the TEPERATURE CONTROL screen section below.

Pressing this button will select the FLUID LEVEL CONTROL screen.  For further information, refer to the FLUID LEVEL CONTROL screen below.

Pressing the “CLOSE” button will close the current screen and return to the previous screen or to the SYSTEM OVERVIEW screen.

- Numerical Display

PROCESS TEMP:  # # # # #

The parameters above are displayed in engineering unit (inches per minute, degrees F, etc,).  They display the actual value of the process parameter (conveyor speed, bath temperature, etc).

- Temperature Control

This screen permits the adjustment of the Temperature / Oscillation Setpoint and  the High and Low Warning windows.   Pressing any of the three fields on the TEMPERATURE CONTROL and OSCILLATION CONTROL screen, (SETPOINT:, WARNING HIGH: or WARNING LOW:) causes the following numeric entry screen to appear:

- Fluid Level Control

This screen permits the adjustment of the Fluid Level Setpoint and  the High and Low Warning windows.   Pressing any of the three fields on the FLUID LEVEL CONTROL screen, (SETPOINT:, WARNING HIGH: or WARNING LOW:) causes the following numeric entry screen to appear:

- PH Control

This screen permits the adjustment of the PH Control, ORP Control, Specific Gravity Control Setpoint and  the High and Low Warning windows.   Pressing any of the three fields on the CONTROL screen, (SETPOINT:, WARNING HIGH: or WARNING LOW:) causes the following numeric entry screen to appear:

- SERVICE MODE

The SERVICE MODE screen, and following two screens are used for factory and field setup by Chemcut personnel only!  They are included in this manual for informational purposes only.

Improper use of these screens can produce improper machine operation resulting in machine damage, site damage, and even personnel injury.

These screens may only be accessed by qualified individuals under the specific direction of Chemcut personnel.

TROUBLESHOOTING / FAULT CODES

# CONTENTS………………………………………………………………………………Page

6.1	General	40

6.2	Malfunctions and Probable Causes	41

6.3	System won't start No indicator lights	41

6.4	System inoperative Message on Message Display	41

6.4.1	Motor Overload	41

6.4.2	Fluid Level	41

6.4.3	Overtemp	41

6.4.4	Lid Open	41

6.4.5	Low Water Pressure	42

6.4.6	Emergency	42

6.4.7	Pump or dryer does not operate	42

6.4.8	Conveyor does not operate	42

6.4.9	Temperature higher than setpoint	42

6.4.10	Temperature lower than setpoint	42

6.4.11	Oscillation inoperative	43

General

# The system has its own diagnostic troubleshooting tool, the Message Display. The Message Display not only displays potential problems (warnings), but also when an unexpected malfunction has occurred.  It not only displays what the malfunction is (i.e. fluid level motor overload, etc.), but also the process station in the system (i.e. Etch, Strip, Rinse etc.).

Malfunctions and Probable Causes

# The following is a list of possible malfunctions and the probable cause.

# MALFUNCTION                     	PROBABLE CAUSE

System won't start No indicator lights

a. Main Power Switch off

b. Start button not pushed

System inoperative Message on Message Display

c. Main power control fuse blown

Motor Overload

a. Motor overload relay tripped

b. Defective motor

Fluid Level

a. Solution is to low or to high

b. Float switch hung up

c. Fluid sensor or switch defective

d. Supply or drain valve defective

Overtemp

a. Cooling system inoperative See 5

b. Heating system inoperative See 6

c. Overtemp probe defective

Lid Open

a. Lid not in place correctly

b. Lid interlock switch defective

Low Water Pressure

a. Water supply turned off or inadequate

b. Pressure switch defective

Emergency

a. Emergency Stop button pushed

b. Emergency Stop button defective

Pump or dryer does not operate

a. Primary fuse(s) blown

b. Motor starter defective

c. Motor defective

d. Clogged filters

Conveyor does not operate

a. Control power fuse blown

b. Broken conveyor gears or coupling

c. Motor and/or gear head defective

d. Conveyor printed circuit assembly defective

e. Interlock condition exists. Conveyor will operate in Runout mode only.

Temperature higher than setpoint

a. Cooling water not turned on

b. Cooling solenoid defective

c. Heater contactor defective

d. RTD probe defective

e. 4 - 20 ma transmitter defective

f. Exothermic chemical reaction

Temperature lower than setpoint

a. Heaters not turned on

b. Heater fuse(s) blown

c. Heater contactor defective

d. RTD probe defective

e. Cooling solenoid defective

f. Cooling coil defective

Oscillation inoperative

a. Control power fuse blown

b. Mechanical linkage loose or disconnected

c. Motor defective

PREVENTATIVE MAINTENANCE

# Contents..…………………..…………………………………………………………….Page

7.1	General	45

7.2	Schedule phases	45

7.2.1	Preventative Maintenance Log	46

7.3	Wear items	50

7.3.1	Gears	50

7.3.2	Bearings	50

7.3.3	Couplings	50

7.3.4	Nozzles	51

7.3.5	Oscillation	51

7.3.6	Manifold packing	51

7.3.7	Manifold adaptors	51

7.4	Consumable parts	52

7.4.1	Lamps	52

7.4.2	Filters	52

7.4.3	Thin material transport	52

7.4.4	Drive belts	52

7.4.5	Probes	52

General

A preventive maintenance program is essential to keep the equipment reliably producing quality product and to ensure that this quality production continues for years into the future.  A preventative maintenance program serves two primary purposes.

The first purpose is to maintain the equipment in peak operating condition.  Sprockets, bearings, pumps, and other moving parts will wear with time.  Periodic inspections are the best way to locate and repair (or replace) worn parts before they cause unwanted down time.  It will also remove crystal deposits that accelerate wear on moving parts.  Chemcut’s history has shown that customers who keep their equipment clean experience much less down time and longer equipment life than those who do not.

The second purpose to avoid unscheduled and unwanted downtime.  By replacing worn items at regularly scheduled shut down times, the equipment overall down time will be reduced.  This will improve scheduling of production time, with better productivity of the equipment as a result.

The combination of equipment in top operating condition and improved productivity are compelling reasons to institute a preventative maintenance program and to maintain this program throughout the life of the equipment.   With a strong focus on preventative maintenance, Chemcut is confident that the equipment will provide a long and profitable life.

Schedule phases

The schedules and procedures for the recommended preventive maintenance program are broken down into phases based on time.   The terms used, and a short description for each of these phases are shown below:

- Daily  (every 8 hours of operation)

- Weekly  (every 40 hours of operation)

- Monthly  (every 160 hours of operation)

- Quarterly  (every 520 hours of operation)

- Semiannually  (every 1040 hours of operation)

- Annually  (every 2080 hours of operation)

- Overhaul  (after 3-5 years of operation)

Preventative Maintenance Log

Any good preventive maintenance program includes the keeping of a P.M. logbook.  Blank sample pages of a logbook are included in the following pages.  Each page is for a one-month period.  Blanks are provided on the sheets to record the system serial number and the month / year.   These P.M. records should be kept in chronological order in a common binder or file.  P.M., maintenance, and service records should be kept for the entire life of the equipment.

The first sample page is for the Daily and Weekly checks to be performed.  It is recommended that the person performing the P.M. check, sign their first initial and last name.  This provides an audit trial to ensure that the P.M. procedure is being performed and is being done correctly.

The second page is very similar to the first, except it is for the remaining P.M. checks, (Monthly, Quarterly, Semi-annually, & Annually).  This second page also includes “Date Scheduled” fields to assist in planning the dates to perform the Monthly, Quarterly, Semi-annual, & Annual maintenance tasks.

Finally; in conjunction with the P.M. log, a Service / Repair Log should be maintained.  This provides another checkpoint as to the actual performance of the system.  This log should contain the date of any malfunction, the corrective action taken, and any problems encountered with the corrective action.

A blank sample of the type of Service / Repair log is also included on the following pages.  A description of the columns follows.

INCIDENT NUMBER - Used for reference purposes.

DATE PROBLEM - Month/day/year problem occurred.

MODULE/PROBLEM - Module that problem occurred in/on and a brief description of the problem.

ACTION TAKEN – A brief description of corrective action that was taken to correct the problem.

DATE/WHO CORRECTED - Month/day/year that corrective action was completed and the name of the individual that conducted the repair.

These P.M. and Service/Repair logs are an important tool to learn from past failures, and to plan future maintenance and service activities.  Proper use of these logs, will result in increased production and will maximize the productive life of the equipment. 
Serial #: _________________     Month/Year: _____/_____

Daily PM Log:  	These procedures are to be conducted daily.  After completing the

procedure, Initial this log under the correct day of the month.

|  | P.M. | 1st | 2nd | 3rd | 4th | 5th | 6th | 7th |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D-1 |  |  |  |  |  |  |  |
| WEEK | D-2 |  |  |  |  |  |  |  |
| 1 | D-3 |  |  |  |  |  |  |  |
|  | D-4 |  |  |  |  |  |  |  |
|  | P.M. | 8th | 9th | 10th | 11th | 12th | 13th | 14th |
|  | D-1 |  |  |  |  |  |  |  |
| WEEK | D-2 |  |  |  |  |  |  |  |
| 2 | D-3 |  |  |  |  |  |  |  |
|  | D-4 |  |  |  |  |  |  |  |
|  | P.M. | 15th | 16th | 17th | 18th | 19th | 20th | 21st |
|  | D-1 |  |  |  |  |  |  |  |
| WEEK | D-2 |  |  |  |  |  |  |  |
| 3 | D-3 |  |  |  |  |  |  |  |
|  | D-4 |  |  |  |  |  |  |  |
|  | P.M. | 22nd | 23rd | 24th | 25th | 26th | 27th | 28th |
|  | D-1 |  |  |  |  |  |  |  |
| WEEK | D-2 |  |  |  |  |  |  |  |
| 4 | D-3 |  |  |  |  |  |  |  |
|  | D-4 |  |  |  |  |  |  |  |
|  | P.M. | 29th | 30th | 31st |  |  |  |  |
|  | D-1 |  |  |  |  |  |  |  |
| WEEK | D-2 |  |  |  |  |  |  |  |
| 5 | D-3 |  |  |  |  |  |  |  |
|  | D-4 |  |  |  |  |  |  |  |

Weekly PM Log:   These procedures are to be completed weekly.  After completing

the procedure, date and initial under the proper procedure number.

|  | W-1 | W-2 | W-3 | W-4 | W-5 | W-6 | W-7 | W-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WEEK 1 |  |  |  |  |  |  |  |  |
| WEEK 2 |  |  |  |  |  |  |  |  |
| WEEK 3 |  |  |  |  |  |  |  |  |
| WEEK 4 |  |  |  |  |  |  |  |  |
| WEEK 5 |  |  |  |  |  |  |  |  |

Serial #: _________________     Month/Year: _____/_____

Monthly PM Log:   These procedures are to be completed monthly.  After completing

the procedure, date and initial under the proper procedure number.

| Procedure | Date Scheduled | Date Completed | Individual who completed P.M. |
| --- | --- | --- | --- |
| M1 |  |  |  |
| M2 |  |  |  |
| M3 |  |  |  |
| M4 |  |  |  |

Quarterly PM Log:   These procedures are to be completed quarterly.  After completing

the procedure, date and initial under the proper procedure number.

| Procedure | Date Scheduled | Date Completed | Individual who completed P.M. |
| --- | --- | --- | --- |
| Q1 |  |  |  |
| Q2 |  |  |  |

Semi-annual PM Log:   This procedure is to be completed semi-annually.  After

completing, date and initial below.

| Procedure | Date Scheduled | Date Completed | Individual who completed P.M. |
| --- | --- | --- | --- |
| S1 |  |  |  |

Annual PM Log:       These procedures are to be completed annually.  After completing

the procedure, date and initial under the proper procedure number.

| Procedure | Date Scheduled | Date Completed | Individual who completed P.M. |
| --- | --- | --- | --- |
| A1 |  |  |  |
| A2 |  |  |  |

Serial #: _________________    Repair and Service Log

| Incident Number | Date Problem | Module/ Problem | Corrective Action Taken | Date / Who Corrected |
| --- | --- | --- | --- | --- |

Wear items

The following items are considered as normal wear items and are not covered under the one-year/2000 hour equipment warranty.  These items should be included in a recommended spare parts package and are to be maintained in the customer’s spare parts inventory.

The replacement frequency of wear items is dependent on a variety of factors, such as, the type of chemistries used, the type of material transported, the conveyor speed, operator skill, and the seriousness with which a preventive maintenance program is established.

Preventive maintenance calls for periodic inspections of the systems components.  Wear items are to be inspected at recommended time intervals and replaced as necessary.  Replacing such parts before they fail will prevent system failure during normal use.

NOTE: Preventive maintenance procedures are based on hours of system operation. You may wish to change the frequency of the inspections after you have gained sufficient experience with your particular system. The following is recommended.

Gears

Gears are to be inspected in accordance with maintenance procedure W-6.

Bearings

Side rail bearings will deteriorate over time, depending on the chemistry encountered, the conveyor speed and hours of operation.  Wear will be obvious, if present.  Check along with procedure W-6.

Couplings

Check the couplings between modules in accordance with maintenance procedure W-6.

Nozzles

Nozzles have a dramatic effect on the process of some materials.  Other processes are very forgiving in respect to clogged and worn nozzles.  Check nozzles in accordance with maintenance procedure D-1.

Oscillation

The moving parts of the oscillation assembly wear and are to be inspected in accordance with maintenance procedure M-5.

Manifold packing

The packing life depends on hours of operation, conveyor speed and the chemistry used.  Inspect in accordance with maintenance procedure M-5.

Manifold adaptors

The manifold adapter life depends on the hours of operation, conveyor speed, and the chemistry used.  Inspect in accordance with maintenance procedure M-5.

Consumable parts

The following items are considered as consumable items and are not covered under the one-year/2000 hour equipment warranty.  These items should be included in a recommended spare parts package and are to be maintained in the customer’s spare parts inventory.

Lamps

Lamps (light bulbs) are considered as consumable items and are to be replaced as needed and inspected in accordance with maintenance procedure S-2.

Filters

Air and water filters are considered as consumable items and are to be replaced as required.  These items are to be inspected in accordance with maintenance procedures W-1 and W-2.

Thin material transport

The fingers and associated parts used in thin material packages are considered as consumable items.  These items are to be inspected in accordance with maintenance procedure W-6.

Drive belts

Belts are considered as consumable items.  These items are to be inspected in accordance with maintenance procedure M-2.

Probes

PH probes and ORP probes will deteriorate over time.  These items are to be calibrated periodically and replaced as necessary.

Maintenance and Adjustment Information

# Contents..………………….…….……………………………………………………….Page

8.1	PREVENTIVE MAINTENANCE CHECKS	55

8.1.1	EVERY 8 HOURS	55

8.1.2	EVERY 40 HOURS	55

8.1.3	EVERY 160 HOURS	55

8.1.4	EVERY 1040 HOURS	56

8.1.5	EVERY 2080 HOURS	56

8.2	MAINTENANCE PROCEDURE – D-1	57

8.2.1	Check nozzle spray pattern	57

8.3	MAINTENANCE PROCEDURE – D-2	58

8.3.1	Clean internal filters	58

8.4	MAINTENANCE PROCEDURE – D-3	59

8.4.1	Check external filter	59

8.5	MAINTENANCE PROCEDURE – D-4	60

8.5.1	Clean exterior of module	60

8.6	MAINTENANCE PROCEDURE – W-1	61

8.6.1	Replace in line water filters	61

8.7	MAINTENANCE PROCEDURE – W-2	62

8.7.1	Replace air filter on dryer	62

8.8	MAINTENANCE PROCEDURE – W-3	63

8.8.1	Check interior of module for cleanliness	63

8.9	MAINTENANCE PROCEDURE – W-4	64

8.9.1	Check PH	64

8.10	MAINTENANCE PROCEDURE – W-5	65

8.10.1	Clean baume stand pipes	65

8.11	MAINTENANCE PROCEDURE – W-6	66

8.11.1	Inspect gear train	66

8.12	MAINTENANCE PROCEDURE – W-7	67

8.12.1	Inspect fluid head	67

8.13	MAINTENANCE PROCEDURE – W-8	68

8.13.1	Inspect flood bar	68

8.14	MAINTENANCE PROCEDURE – M-1	69

8.14.1	Check conveyor drive belts	69

8.15	MAINTENANCE PROCEDURE – M-2	70

8.15.1	Check alignment of spray tubes	70

8.16	MAINTENANCE PROCEDURE – M-3	71

8.16.1	Check plumbing for leaks	71

8.17	MAINTENANCE PROCEDURE – M-4	72

8.17.1	Inspect oscillation system	72

8.18	MAINTENANCE PROCEDURE – Q-1	73

8.18.1	Inspect oscillation system	73

8.19	MAINTENANCE PROCEDURE – Q-2	74

8.19.1	Inspect control panel	74

8.20	MAINTENANCE PROCEDURE – S-1	75

8.20.1	Inspect junction boxes	75

8.21	MAINTENANCE PROCEDURE – A-1	76

8.21.1	Inspect pumps	76

8.22	MAINTENANCE PROCEDURE – A-2	78

8.22.1	Inspect blower motor	78

8.23	MAINTENANCE PROCEDURE – O-1	79

8.23.1	Overhaul Equipment	79

PREVENTIVE MAINTENANCE CHECKS

The following checks are to be performed on a predetermined schedule based on hours of operation of the system. You may need to adjust the frequency of the inspections after you have gained sufficient experience with your particular system.  The following is a quick reference list of checks.  Detailed maintenance procedures follow this list.

EVERY 8 HOURS

EVERY 8 HOURS

CHECK NOZZLES

CHECK INTERNAL FILTERS

CHECK EXTERNAL FILTERS

CLEAN EXTERIOR OF MODULES

EVERY 40 HOURS

EVERY 40 HOURS

REPLACE WATER FILTERS

REPLACE AIR FILTERS

CLEAN INTERIOR OF MODULES

CHECK PH

CLEAN BAUME STAND PIPE

INSPECT CONVEYOR GEARS

INSPECT FLUIDHEADS

INSPECT FLOODBARS

EVERY 160 HOURS

EVERY 160 HOURS

CHECK CONVEYOR BELTS

CHECK SPRAY TUBE ALIGNMENT

CHECK PLUMBING

INSPECT OSCILLATION

EVERY 1040 HOURS

EVERY 1040 HOURS

CHECK INTERLOCKS

INSPECT CONTROL PANEL

EVERY 2080 HOURS

EVERY 2080 HOURS

INSPECT JUNCTION BOXES

INSPECT PUMPS

INSPECT BLOWERS

MAINTENANCE PROCEDURE – D-1

Check nozzle spray pattern

PROCEDURE:  Check nozzle spray pattern - clean nozzles as necessary.

FREQUENCY:  Every 8 hours of operation.

TOOLS REQUIRED:  Screwdriver, needle nosed pliers, wrench, air hose.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .03 man hours to inspect.  .03 man hours per clogged nozzle.

PROCEDURE STEPS:

1.	Visually inspect the spray pattern by looking through the cover and/or doors of the module.  It may be necessary to alternately turn off the upper and lower sprays to see the pattern.

If clogged nozzles have been observed:

2.	Turn off the module.

3.	Remove cover, or, door to gain access to the clogged nozzle.

4.	Remove the clogged nozzle.

5.	Remove the insert from the nozzle.

6.	Blow compressed air through the nozzle to clean.  Soak the nozzle in cleaning solution first, if required.)  Do not use wire or scrape the nozzle to remove debris as this will damage the orifice.

7.	Replace the nozzle(s) and cover(s).

NOTE:  If nozzles are frequently clogged, check for contaminated chemistry, clogged filters, etc.

MAINTENANCE PROCEDURE – D-2

Clean internal filters

PROCEDURE:  Clean internal filters.

FREQUENCY:  Every 8 hours of operation.

TOOLS REQUIRED:  Screwdriver.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .03 man hours.

PROCEDURE STEPS:

1.	Turn off module.

2.	Remove the sump cover, door, etc., as required to gain access to the filter.

3.	Remove the filter from the module.

4.	Clean the filter, removing any collected residue.

5.	Replace the filter.

6.	Replace the sump cover, door, etc.

MAINTENANCE PROCEDURE – D-3

Check external filter

PROCEDURE:  Check external filter.

FREQUENCY:  Every 8 hours of operation.

TOOLS REQUIRED:  Screwdriver.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .01 man hours.  .05 if filter is changed.

PROCEDURE STEPS:

1.	Visually inspect the filter.

If Required:

2.	Remove the dirty filter.

3.  	Install a new filter.

NOTE:  The frequency of filter replacement may require it be changed either more or less often.

MAINTENANCE PROCEDURE – D-4

Clean exterior of module

PROCEDURE:  Clean exterior of module.

FREQUENCY:  Every 8 hours of operation.

TOOLS REQUIRED:  Soft bristle scrub brush, water hose, air hose.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .03 man hours.

PROCEDURE STEPS:

1.	Turn off module.

2.	Clean the exterior of the module using brush and water.  Remove all crystal deposits

3.	Blow off excess water with the air hose.

NOTE:  Avoid spraying water directly on pump motors and electrical connections.

MAINTENANCE PROCEDURE – W-1

Replace in line water filters

PROCEDURE:  Replace in line water filters (as required.)

FREQUENCY:  Every 40 hours of operation.

TOOLS REQUIRED:  None.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .01 man hours to inspect.  .03 man hours to replace.

PROCEDURE STEPS:

1.	Visually inspect the filter cartridge.

2.	If the filter is to be replaced, turn off the input water valve.

3.	Remove the filter.

4.	Replace the filter.

NOTE: The frequency of filter replacement may require it be changed either more or less often.

MAINTENANCE PROCEDURE – W-2

Replace air filter on dryer

PROCEDURE:  Replace air filter on dryer (as required.)

FREQUENCY:  Every 40 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a  respirator if a high concentration of fumes  is present.

TIME REQUIRED:  .02 man hours.

PROCEDURE STEPS:

- Turn off module.

- Remove the air filter and inspect - replace as required.

- Reinstall the filter.

NOTE: The frequency of filter replacement may require it be changed either more or less often.

MAINTENANCE PROCEDURE – W-3

Check interior of module for cleanliness

PROCEDURE:  Check interior of module for cleanliness (clean as required.)

FREQUENCY:  Every 40 hours of operation.

TOOLS REQUIRED:  Soft bristle scrub brush, bottle brush, water hose, air hose.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .1 man hours

PROCEDURE STEPS:

1.	Turn off system.

2.	Drain solution from module.

3.	Remove lids, doors, to gain access to inside of the module.

- Rinse out interior of module.  Remove all crystal growth with a scrub brush.

5.	Clean deposits from doors, lids and sealing surfaces.

6.	Clean gear train with a bottle brush.

7.	Rinse out module.

8.	Replace doors and lids.

NOTE:  The frequency of cleaning depends on the type and condition of the chemistry used in the module.

MAINTENANCE PROCEDURE – W-4

Check PH

PROCEDURE:  Check PH.  Use buffer solution to check accuracy of probes.

FREQUENCY:  Every 40 hours of operation.

TOOLS REQUIRED:  Buffer solution.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .15 man hours

PROCEDURE STEPS:

1.	Turn off module.

2.	Remove ph probe from its housing assembly.

3.	Wash probe with deionize water.

4.	Immerse ph probe in a buffer solution of a known value.

5.	Standardize if required.

6.	Reinstall probe into its housing.

MAINTENANCE PROCEDURE – W-5

Clean baume stand pipes

PROCEDURE:  Clean baume stand pipes - readjust flow, as required.

FREQUENCY:  Every 40 hours of operation.

TOOLS REQUIRED:  Commercial glass cleaner and soft rags.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .03 man hours.

PROCEDURE STEPS:

1.	Turn off module.

2.	Remove the baume standpipe and clean with a soft rag.

3.	Reinstall the baume standpipe.

4.	Turn on the module and readjust flow, as required.

MAINTENANCE PROCEDURE – W-6

Inspect gear train

PROCEDURE:  Inspect gear train.  Replace worn gears, as required.

FREQUENCY:  Every 40 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .03 man hours.

PROCEDURE STEPS:

1.	Turn off module.

2.	Remove covers/doors, to gain access to module gear train.

3.	Inspect all gears, checking for proper gear mesh and worn teeth.

4.	Notify maintenance personnel of any gear wear or other flaws.

NOTE:  Obvious flaws need to be corrected right away.  Minor irregularities may wait until the next scheduled PM.  All flaws should be reported to maintenance personnel when observed.

MAINTENANCE PROCEDURE – W-7

Inspect fluid head

PROCEDURE:  Inspect fluid head.  Clean if necessary.

FREQUENCY:   Every 40 hours of operation.

TOOLS REQUIRED:   Screwdriver, adjustable wrench, low pressure air, vacuum suction

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate

protective clothing (gloves, apron, etc.) when working near modules containing hazardous

chemistry.  Use a respirator if a high concentration of fumes is present..

TIME REQUIRED:   1.0 man hours

PROCEDURE STEPS:

1.	While machine is in operation, look through covers and see if any fluid heads are clogged.  Note the location of any that are.

2.	Turn off the module.

3.	Remove covers.

4.	Use clean compressed air, aimed at the injector holes with vacuum suction to the plenum fitting.  If necessary, use a narrow diameter quill to clear debris from the injector holes, being careful not to deform the orifice of the holes.

5.	Replace covers.

MAINTENANCE PROCEDURE – W-8

Inspect flood bar

PROCEDURE:  Inspect flood bar.  Clean if necessary.

FREQUENCY:   Every 40 hours of operation..

TOOLS REQUIRED:   Screwdriver, adjustable wrench, low pressure air.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate

protective clothing (gloves, apron, etc.) when working near modules containing hazardous

chemistry.  Use a respirator if a high concentration of fumes is present..

TIME REQUIRED:   1.0 man hours

PROCEDURE STEPS:

1.	While machine is in operation, look through covers and see if  any  flood bars are clogged.  Note the location of any that are.

2.	Turn off the module.

3.	Remove covers.

4.	Remove either blade of clogged flood bar to clean slot of flood bar.

5.	Replace covers.

NOTE:  Do not use a piece of wire or any sharp object to clean flood bar.  This will deform the flood bar slot and change the flooding characteristics of the flood bar.

MAINTENANCE PROCEDURE – M-1

Check conveyor drive belts

PROCEDURE:  Check conveyor drive belts.  Adjust, or, replace as necessary.

FREQUENCY:  Every 160 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver, adjustable wrench.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .05 man hours to inspect.  .1 man hours to replace.

PROCEDURE STEPS:

1.	Turn off the module.

2.	Remove the cover to gain access to the drive belt.

3.	Check the belt and sprocket for alignment and wear.  Replace the belt, if necessary.

4.	Reassemble the unit.

MAINTENANCE PROCEDURE – M-2

Check alignment of spray tubes

PROCEDURE:  Check alignment of spray tubes (developers and etchers,) adjust as necessary.

FREQUENCY:  Every 160 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver, adjustable wrench.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .08 man hours.

PROCEDURE STEPS:

1.	Remove the module doors.

2.	Check the alignment of the spray tubes.

3.	Make any adjustments, as required.

4.	Replace the module doors.

*  This procedure may be accomplished in conjunction with M-4 (Inspect oscillation system).

MAINTENANCE PROCEDURE – M-3

Check plumbing for leaks

PROCEDURE:  Check plumbing for leaks.  Tighten as necessary.

FREQUENCY:  Every 160 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver, thread tape, strap wrench.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .04 man hours.

PROCEDURE STEPS:

1.	With the module running, inspect for leaks of the plumbing.

- Unusual sounds.

- Low pump pressure.

- Unusual spray pattern on door or top cover.

2.	Locate the source of any leaks.

3.	Turn off the module and repair any leaks.

4.	Turn the module on and re-inspect for any additional leaks.

MAINTENANCE PROCEDURE – M-4

Inspect oscillation system

PROCEDURE:  Inspect oscillation system.  Check shoulder bolts of the oscillation for

tightness.  Inspect manifold seals for leakage.

FREQUENCY:  Every 160 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver, adjustable wrench.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .06 man hours.

PROCEDURE STEPS:

1.	With the module operating:

- Check for proper spray pressure.

- Check for any unusual sounds.

- Check for unusual spray pattern on the doors, or, top cover.

- Turn the module off and remove the module doors.

3.	Check the shoulder bolts in the oscillation pivot blocks for tightness.

4.	Check the oscillation movement for smooth operation.

- Check the oscillation drive motor v-seal and packing for leakage.

6.	Reassemble the module.

MAINTENANCE PROCEDURE – Q-1

Inspect oscillation system

PROCEDURE:  Check interlock functions (door, fluid level, etc.) Repair/adjust as necessary.  Check emergency stop switches.

FREQUENCY:  Every 1040 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves,  apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes  is present.

TIME REQUIRED:  .3 man hours.

PROCEDURE STEPS:

1.	Turn off pump (if applicable) and check lid interlock by removing the lid.  Insure that interlock occurs.  Repair/replace defective interlocks.

2.	Remove sump lid to gain access to fluid level interlocks.  Move  switches up and down to check high and low level interlocks.  Insure that the proper interlock occurs.

3.	Check pressure switches.  Turn off air supply and insure that interlock occurs.  Turn off water supply and insure that interlock occurs.

4.	Reassemble the module.

*  This procedure may be accomplished in conjunction with S-2.  Inspect control panel and functions.

MAINTENANCE PROCEDURE – Q-2

Inspect control panel

PROCEDURE:  Inspect control panel (s.)  Inspect all functions.  Replace bulbs, switches, etc., as necessary.

FREQUENCY:  Every 1040 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .1 man hours.

PROCEDURE STEPS:

1.	Check all operator controls for proper function.

- Turn pumps, conveyor, etc. on/off.

- Ensure that the proper indicator lights are illuminated.

- Check all interlocks for proper function.

2.	Check all functions of the system.  Start the system and check that all operations occur properly in both the manual and auto modes of operation.

3.	Replace any burned out indicator lights or defective switches.

*  This procedure may be accomplished in conjunction with S-1 (check interlock functions).

MAINTENANCE PROCEDURE – S-1

Inspect junction boxes

PROCEDURE:  Inspect junction boxes.  Check for leakage and/or corrosion.  Repair or replace as necessary (terminals, gasket material, etc.)

FREQUENCY:  Every 2080 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver, adjustable wrench.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  .1 man hours.

PROCEDURE STEPS:

1.	Turn off power to the system.

2.	Remove the junction box cover.

3.	Inspect the box for signs of corrosion.

- Check the gasket material.  Replace as necessary.

- Check for leakage.

- Check for corroded or loose terminals.

4.	Tighten and/or repair wiring as necessary.

5.	Replace the junction box cover.

MAINTENANCE PROCEDURE – A-1

Inspect pumps

PROCEDURE:  Inspect pumps.  Remove pumps and check for damaged components.  Replace parts as necessary.

FREQUENCY:  Every 2080 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver, adjustable wrench.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a respirator if a high concentration of fumes is present.

TIME REQUIRED:  One man hour.

PROCEDURE STEPS:

1.	Turn off power.

2.	Remove the pump.

3.	Check for damaged and/or worn components.  Replace as necessary.

4.	Inspect motor for corrosion.  (If badly corroded, replacement should be considered.)

5.	Reassemble pump.  Set impeller clearance to back plate at 045".

6.	Reassemble module.

MAINTENANCE PROCEDURE – A-2

Inspect blower motor

PROCEDURE:  Inspect blower motor.

FREQUENCY:  Every 2080 hours of operation.

TOOLS REQUIRED:  Flat blade screwdriver.

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a  respirator if a high concentration of fumes is present.

TIME REQUIRED:  .05 man hours.

PROCEDURE STEPS:

1.	With the blower running, listen for any unusual sounds.

2.	Turn off module.

3.	Remove the cover to gain access to the blower motor.

4.	Inspect the motor.  Ensure that the shaft turns freely.  Resistance to turning could indicate worn bearings.  Replace as necessary.

5.	Inspect the belts for wear.

6.	Reassemble the module.

MAINTENANCE PROCEDURE – O-1

Overhaul Equipment

PROCEDURE:  Overhaul Equipment

FREQUENCY:  Every 3 to 5 years  of operation.

TOOLS REQUIRED:  as required

SAFETY REQUIREMENTS:  Eye protection is to be worn at all times.  Wear appropriate protective clothing (gloves, apron, etc.,) when working near modules containing hazardous chemistries.  Use a  respirator if a high concentration of fumes is present.

TIME REQUIRED:  as required

PROCEDURE STEPS:

Every three to five years an overhaul should be considered.  This would consist of first doing an in-depth cleaning of the equipment.  Then the conveyor should be overhauled.  Gears, bearings, conveyor siderail inserts, conveyor rods, conveyor wheels, and drive motors should be checked.  Any worn or corroded items should be replaced.

Then plumbing systems should be checked.  Replace all nozzles, as the orifice in the nozzle will wear slowly.  Check all spray tubes, pipes, pumps, valves, etc.  Replace or repair leaky fittings or warped spray tubes.  Check the spray tube bushings for wear and replace if necessary.  Check pumps and motors.  Replace all worn or corroded items.

Then check all junction boxes, control panels, etc.  Replace all corroded connections and bad electrical parts.  Replace any cracked or warped covers.

If you would have questions as to what work would need to be done, Chemcut will gladly audit the equipment, provide an estimate to overhaul the equipment, and explain all items.

Parts & Spare Parts

# Contents……………………………….………………………………………  Page

9.1	PLE634       Load & Drive	82

9.1.1	PLE634M         PLE634,MEC PARTS,P-SIGMA	82

9.1.2	CNVR,ROLL,PLE634,LR,C60-500	83

9.1.3	ROLL,60,2X2,SOLID,LWR,TPN/T	85

9.1.4	DRIVE,PLE634,140IPM,TI,C60-500	87

9.2	PIA634        Conveyor	89

9.2.1	PIA634M         PIA634,MEC PARTS,P-SIGMA	89

9.2.2	CNVR,ROLL,PIA634,LR,C60-500	90

9.2.3	ROLL,60,2X2,SOLID,LWR,TPN/T	92

9.2.4	VENT INSTL,P-SIGMA,60"	94

9.3	POS607        Ferric Chloride Etch	96

9.3.1	POS607M-1       POS607,MEC PARTS,NO/AIR SEAL	96

9.3.2	POS607 MOD ASSY,NO/AIR SEAL	96

9.3.3	MODULE INT,POS607,TI,N/AIRSEAL	97

9.3.4	DRIVE INSTL,OSCILLATION,POS607 503097 No Drawing	98

9.3.5	SPRAYRACK INSTL,POS607,(40-PG)	100

9.3.6	SPRAYRACK ASSY,UP/LEFT,POS607	102

9.3.7	SPRAYRACK ASSY,LOW/LEFT,POS607	104

9.3.8	SPRAYRACK ASSY,UP/RIGHT,POS607	106

9.3.9	SPRAYRACK ASSY,LOW/RHGT,POS607	108

9.3.10	CNVR,ROLL,POS607,LR,N/OUT ROLL	110

9.3.11	ROLL,60,2X2,SOLID,TPN/TI*	112

9.3.12	ROLL,60,2X2,SOLID,LWR,TPN/T	114

9.3.13	ROD,60,2X2,LWR ONLY,GRAPHITE	116

9.3.14	PLUMBING,POS607	118

9.3.15	PLUMBING,POS607,LOWER RACK	119

9.3.16	PLUMBING,POS607,UPPER RACK	121

9.3.17	PLUMBING,POS607,DRAIN	123

9.3.18	PUMP SUBASSY,P-55J-C1	124

9.3.19	Pump Specifications	126

9.3.20	COOLING,POS607	127

9.3.21	GAUGE INSTL,POS607,0-60 (40)	130

9.3.22	GAUGE INSTL,PRESS,PEM,(2)	132

9.3.23	NOZZLE INSTL,EDGE ETCH	134

9.3.24	HEAT,6X6KW,440-480V,TI,M BLD	136

9.3.25	JCT BOX INSTL,AIRGAP,POS06	138

9.3.26	RECIRC PUMP,(2)POS,3HP,W/FEEDS	140

9.3.27	INTERCONNECT,SUMP,POS (4-EA)	142

9.3.28	SUPPLY,PVC SOL,1/2",POS	144

9.4	POS607        Ferric Chloride Etch	145

9.4.1	POS607M-1       POS607,MEC PARTS,NO/AIR SEAL	145

9.4.2	POS607 MOD ASSY,NO/AIR SEAL	145

9.4.3	MODULE INT,POS607,TI,N/AIRSEAL	146

9.4.4	DRIVE INSTL,OSCILLATION,POS607 503097 No Drawing	147

9.4.5	SPRAYRACK INSTL,POS607,(40-PG)	149

9.4.6	SPRAYRACK ASSY,UP/LEFT,POS607	151

9.4.7	SPRAYRACK ASSY,LOW/LEFT,POS607	153

9.4.8	SPRAYRACK ASSY,UP/RIGHT,POS607	155

9.4.9	SPRAYRACK ASSY,LOW/RHGT,POS607	157

9.4.10	CNVR,ROLL,POS607,LR,N/OUT ROLL	159

9.4.11	ROLL,60,2X2,SOLID,TPN/TI*	161

9.4.12	ROLL,60,2X2,SOLID,LWR,TPN/T	163

9.4.13	ROD,60,2X2,LWR ONLY,GRAPHITE	165

9.4.14	PLUMBING,POS607	167

9.4.15	PLUMBING,POS607,LOWER RACK	168

9.4.16	PLUMBING,POS607,UPPER RACK	170

9.4.17	PLUMBING,POS607,DRAIN	172

9.4.18	PUMP SUBASSY,P-55J-C1	173

9.4.19	Pump Specifications	175

9.4.20	COOLING,POS607	176

9.4.21	GAUGE INSTL,POS607,0-60 (40)	180

9.4.22	GAUGE INSTL,PRESS,PEM,(2)	182

9.4.23	NOZZLE INSTL,EDGE ETCH	184

9.4.24	HEAT,6X6KW,440-480V,TI,M BLD	186

9.4.25	JCT BOX INSTL,AIRGAP,POS06	188

9.4.26	RECIRC PUMP,(2)POS,3HP,W/FEEDS	190

9.4.27	INTERCONNECT,SUMP,POS (4-EA)	192

9.4.28	SUPPLY,PVC SOL,1/2",POS	194

9.5	PIA634        Conveyor	195

9.5.1	PIA634M         PIA634,MEC PARTS,P-SIGMA	195

9.5.2	CNVR,ROLL,PIA634,LR,C60-500	196

9.5.3	ROLL,60,2X2,SOLID,LWR,TPN/T	198

9.5.4	VENT INSTL,P-SIGMA,60"	200

9.6	PRE634        Unload & Drive	202

9.6.1	PRE634M         PRE634,MEC PARTS,P-SIGMA	202

9.6.2	CNVR,ROLL,PRE634,LR,C60-500	203

9.6.3	ROLL,60,2X2,SOLID,LWR,TPN/T	205

9.6.4	DRIVE,PRE634,140IPM,TI,C60-500	208

9.7	General Parts Ordering Instructions	211

9.8	Parts Ordering Call 1-800-243-6288	213

9.9	Recommended Spare Parts	214

PLE634       Load & Drive

PLE634M         PLE634,MEC PARTS,P-SIGMA

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	 QUANTITY

1       503184          CHAMBER,PLE634                          			1.

3       502770          COVER,SIDERAIL,FRNT,PRE334              		1.

7       502771          COVER,SIDERAIL,REAR,PRE334              		1.

5       503188          PLENUM,PLE634                           			1.

6       131644          COVER INSTL,FRONT,33.882                		1.

136187          LABELS,SIGMA,TLE\TRE,STD                		1

503078          DRIVE,PLE634,140IPM,TI,C60-500          		1

503122          CNVR,ROLL,PLE634,LR,C60-500             		1

503164          DRAIN BACK,PLE/PRE-PIA-POS,1"           		1

CNVR,ROLL,PLE634,LR,C60-500

#### 503122          CNVR,ROLL,PLE634,LR,C60-500

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       501492          SIDERAIL,REAR,21 ROD,PIA34              			1.

2       501493          SIDERAIL,FRONT,21 ROD,PIA34             			1.

3       023353          PIN,GRAPHITE,1/4ODX1LG                  			2.

4       500932          STRUT,P-SIGMA,STD,TI  *                 			5.

5       500933          STRUT,P-SIGMA,L-R,TI                    				1.

6       502994          SHAFT,LINE,TI,16-11/16                  				1.

7       502995          SHAFT,LINE,TI,18                        				1.

8       501024          GEAR,MITER,13T,PVDF,.929,9P  *          			1.

9       501025          GEAR,MITER,13T,PVDF,1.500,9P *          			5.

10       501026          GEAR,MITER,13T,PVDF,1.662,9P           			12.

11       501060          GEAR,MITER,13T,PVDF,1.474,9P *          			1.

20       501065          PLATE,SKID,DRIVELINE         *          			1.

21       003615          SCREW,PAN,SST18-8,10-32 X 1/4"          			2.

23       503130          ROLL,60,2X2,SOLID,LWR,TPN/TI            			7.

ROLL,60,2X2,SOLID,LWR,TPN/T

#### 503130          ROLL,60,2X2,SOLID,LWR,TPN/T

I

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	              QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				1.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              			1.

10       141546          CLIP,RETAINER,PPL                       				1.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

DRIVE,PLE634,140IPM,TI,C60-500

503078          DRIVE,PLE634,140IPM,TI,C60-500

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       502383          MOUNT,MOTOR,DRIVE,PIA*34                			  1.

2       067264          PVC,1/2 X  3                        					34.000

2       067264          PVC,1/2 X  3                        					34.000

3       035566          SCREW,FLAT,SST316,1/4-20X1             			  4.0

4       502361          PLATE,MTG,TENSION ADJ,P-S,TI            			  1.

5       003019          BOLT,HEX,TI,5/16-18X3/4                 			  3.

6       003055          WASHER,FLAT,TI,5/16"NOM                 			  3.

7       502362          PLATE,TENSION ADJUST,P-SIG,TI           			  1.

8       114593          REDUCER,SPEED,60:1                     			  1.0

9       105659          MOTOR,DC,1/4 HP,TACH-7V/K,CE *         			  1.0

10       003019          BOLT,HEX,TI,5/16-18X3/4                 			  4.

11       003055          WASHER,FLAT,TI,5/16"NOM                 			  4.

12       013155          WASHER,LOCK,SST316,5/16"NOM            		  4.0

13       100128          BOLT,HEX,TI,1/4-20X3                    				  2.

15       003056          WASHER,FLAT,TI,1/4 NOM                  			  2.

16       026866          WASHER,LOCK,SST316,1/4"NOM             			  2.0

17       003032          NUT,HEX,TI,1/4-20                       				  2.

18       087396          SPROCKET,BELT,HTD,BLK,5/8ID,24         			  1.0

19       170135          KEY,SQ TI,3/16X3/4LG                    				  1.

20       080853          BELT,HTD,1040-8M-20                    			   	  1.0

21       501087          GEAR,DRIVE,PVDF,9P,1.662,24T           			  1.0

22       501085          SUPPORT,DRIVE,SIDERAIL,P-SIGMA         		  2.0

23       501229          WASHER,DOMED,SST303,5/16-NOM            		  2.

26       503082          PAN,PLE634,OUTPUT DRIVE                 			  1.

27       501230          COVER/BASE PAN SUPPORT,P-SIGMA         		  1.0

28       137354          SHEET,PVC,EUROGRY,1/4X48X100IN        		24.0

29       069565          PVC,1/2 X     1/2                     				24.0

30       067246          PVC,1/2 X  6                        					65.750

31       069686          PVC,1/2 X  1                        					65.750

31       069686          PVC,1/2 X  1                        					65.750

PIA634        Conveyor

PIA634M         PIA634,MEC PARTS,P-SIGMA

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

0       503183          CHAMBER,PIA634                          				1.

3       750034          COVER,SIDERAIL,REAR,PIA334              			1.

4       750035          COVER,SIDERAIL,FRONT,PIA334             			1.

5       503159          PLENUM,PRE634                           				1.

5       503188          PLENUM,PLE634                           				1.

6       131644          COVER INSTL,FRONT,33.882                			1.

7       503158          MOD.INT.PARTS,P-SIGMA,60,TI             			1.

136186          LABELS,SIGMA,TIA,STD                    			1

503127          CNVR,ROLL,PIA634,LR,C60-500             			1

503181          DRIVE,PIA634,NO-DRIVE,PAN ONLY          		1

503194          VENT INSTL,P-SIGMA,60"                  			1

CNVR,ROLL,PIA634,LR,C60-500

#### 503127          CNVR,ROLL,PIA634,LR,C60-500

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       501492          SIDERAIL,REAR,21 ROD,PIA34              			1.

2       501493          SIDERAIL,FRONT,21 ROD,PIA34             			1.

3       023353          PIN,GRAPHITE,1/4ODX1LG                  			2.

4       500932          STRUT,P-SIGMA,STD,TI  *                 			5.

5       500933          STRUT,P-SIGMA,L-R,TI                    				1.

6       501508          SHAFT,LINE,TI,16-9/16                   				1.

7       502995          SHAFT,LINE,TI,18                        				1.

8       501024          GEAR,MITER,13T,PVDF,.929,9P  *          			1.

9       501025          GEAR,MITER,13T,PVDF,1.500,9P *          			5.

10       501026          GEAR,MITER,13T,PVDF,1.662,9P           		          14.

11       501027          GEAR,MITER,13T,BLIND,PVDF,9P *          			1.

27       503130          ROLL,60,2X2,SOLID,LWR,TPN/TI            			7.

ROLL,60,2X2,SOLID,LWR,TPN/T

#### 503130          ROLL,60,2X2,SOLID,LWR,TPN/T

I

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	              QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				1.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              			1.

10       141546          CLIP,RETAINER,PPL                       				1.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

VENT INSTL,P-SIGMA,60"

503194          VENT INSTL,P-SIGMA,60"

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		      QUANTITY

0       503159          PLENUM,PRE634                      				0.00000

0       503188          PLENUM,PLE634                      					0.00000

1       023936          COLLAR,VENT,PVC,MOLDED,4"               			2.

2       011607          DAMPER ASSY,PVC,4"ID                    			2.

3       049817          ADAPTER,VENT,PVC,MOLDED,4"              			2.

4       503195          BAFFLE ASSY,PLENUM,60,P-SIG             			1.

5       002711          SCREW,RD,TI,1/4-20 X 3/4"               				8.

POS607        Ferric Chloride Etch

POS607M-1       POS607,MEC PARTS,NO/AIR SEAL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	 QUANTITY

0       503094          POS607 MOD ASSY,NO/AIR SEAL             		1.

0       503095          MODULE INT,POS607,TI,N/AIRSEAL          		1.

POS607 MOD ASSY,NO/AIR SEAL

#### 503094          POS607 MOD ASSY,NO/AIR SEAL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       503056          CHAMBER,POS607,NO/AIR SEAL              			1.

0       503105          PLUMBING,POS607                         				1.

3       503106          SPRAYRACK INSTL,POS607,(40-PG)          		1.

0       503097          DRIVE INSTL,OSCILLATION,POS607          		2.

0       503099          COOLING,POS607                          				2.

0       503107          JCT BOX INSTL,AIRGAP,POS607             			1.

0       131577          COVER INSTL,FRONT,TOS*06                			1.

0       144090          COVER INSTL,OSC,OS                      			2.

0       501748          COVER,SUMP ACCESS/SAMPLE,POS            		1.

0       501749          COVER,SUMP ACCESS,POS                   			2.

0       600404          PLUG,PORT,PVC,XLI                       				1.

0       170088          GAUGE INSTL,PRESS,PEM,(2)               			2.

136179          LABELS,SIGMA,TOS,STD                    			1

502809          BAG,FILTER,80MIC,PLYESTR,(2)            			1

502817          RECIRC PUMP,(2)POS,3HP,W/FEEDS          		1

503113          GAUGE INSTL,POS607,0-60 (40)            			1

503126          HEAT,6X6KW,440-480V,TI,M BLD            			1

503129          CNVR,ROLL,POS607,LR,N/OUT ROLL          		1

503135          PUMP,2X7.5,P-55CJ-CI,TI IMPELL          			1

503136          NOZZLE,1.5GPM,073711,SCRD(140)          		1

503137          INTERCONNECT,SUMP,POS (4-EA)            		1

503156          SUPPLY,PVC SOL,1/2",POS                 			1

503223	NOZZLE INSTL,EDGE ETCH				1

MODULE INT,POS607,TI,N/AIRSEAL

503095          MODULE INT,POS607,TI,N/AIRSEAL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	 QUANTITY

0       503108          DOOR INSTL,FRONT,POS*07,N/AIRS          		1.

0       503192          DOOR INSTL,REAR,POS*07,N/AIRS           		1.

0       503109          LID INSTL,TOP POS607,N/AIRSEAL          		1.

0       503096          GASKET,CNVR,P-SIGMA,60,TPE55A           		1.

0       503104          ANGLE,OPENING,P-SIGMA,60,TI             		4.

0       141115          BOLT,HEX,TI,1/4-20X1-1/4               		         58.

0       003032          NUT,HEX,TI,1/4-20                      		         58.

0       142534          PIN,GRAPHITE,1/4ODX3/4LG                		2.

0       143787          LABEL,UPPER,INPUT                       			1.

0       143788          LABEL,UPPER,OUTPUT                      		1.

0       143789          LABEL,LOWER,INPUT                       			1.

0       143790          LABEL,LOWER,OUTPUT                      		1.

0       049817          ADAPTER,VENT,PVC,MOLDED,4"              		2.

0       501035          PLATE,CONN,FRONT,P-SIGM,TI              		2.

0       501036          PLATE,CONN,REAR,P-SIGMA,TI              		2.

DRIVE INSTL,OSCILLATION,POS607	503097 No Drawing

503097          DRIVE INSTL,OSCILLATION,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       144240          MOTOR,GEAR,DC,1/4HP,68RPM,RITE          		1.

2       144630          CLAMP ASSY,MOTOR,OSC,PVC,TOS            		1.

3       003024          BOLT,HEX,TI,1/4-20X1                    				4.

4       003056          WASHER,FLAT,TI,1/4 NOM                  			4.

5       501596          BUSHING,UPPER,OSC,POS                   			1.

6       069565          PVC,1/2 X     1/2                   				         12.000

7       501718          BEARING,LOWER,OSC,UHMW                  			1.

8       031243          PIN,TI,.125ODX1LG                       				2.

9       144243          COUPLING,DRIVE,OSC,OS                   			1.

10       148916          KEY,SQ TI,3/16X1-1/2LG                  				1.

11       029163          SETSCREW,TI,1/4-20X1/4",CUP PT          			2.

12       503085          PLATE,SEAL,OSCILLATION,POS,MOD          		1.

12       503092          BRACKET,SHAFT,OSC,LWR,POS607       			0.00000

13       159857          SEAL,O-RING,2-241,EPDM,BLACK            			2.

14       002711          SCREW,RD,TI,1/4-20 X 3/4"               				4.

15       082298          SEAL,V-RING,CVU-1220,PTFE               			1.

16       148952          CAP,BEARING,PVC                         				1.

17       502731          SHAFT ASSY,OSC,POS                      			1.

18       501080          CAM,OSCILLATION,POS                     			2.

19       003008          BOLT,HEX,TI,3/8-16X1-1/2                				2.

20       003012          NUT,HEX,TI,3/8-16                       				2.

SPRAYRACK INSTL,POS607,(40-PG)

#### Lower Racks Shown

503106          SPRAYRACK INSTL,POS607,(40-PG)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

0       503114          SPRAYRACK ASSY,UP/LEFT,POS607           		1.

0       503141          SPRAYRACK ASSY,LOW/LEFT,POS607          		1.

0       503151          SPRAYRACK ASSY,UP/RIGHT,POS607          		1.

0       503169          SPRAYRACK ASSY,LOW/RHGT,POS607          		1.

5       503120          PIN,RACK SUPPORT ROLLER                 			8.

6       503119          ROLLER,RACK SUPPORT                    		         16.

7       130994          WASHER,HDPE,.380IDX1ODX1/16THK         	         16.

8       503121          COLLAR,LOCK,RACK ROLLER                 			8.

SPRAYRACK ASSY,UP/LEFT,POS607

503114          SPRAYRACK ASSY,UP/LEFT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

1       503115          PLATE,TOP,UPPER RACK,LFT,40 NZ          		1.

2       503116          PLATE,BOT,UPPER RACK,LFT,40 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       		         50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                			         10.

12       503128          BLOCK,ROLLER MTG,OFFSET,POS607          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            			         10.

17       503132          BRACE,SPRAYRACK,UPR LFT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503138          BRACE,SPRAYRACK,TI,14.250"             		         20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              			       115.

26       003032          NUT,HEX,TI,1/4-20                     			       115.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           		         10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            		         10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

SPRAYRACK ASSY,LOW/LEFT,POS607

503141          SPRAYRACK ASSY,LOW/LEFT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

1       503142          PLATE,TOP,LOWER RACK,LFT,40 NZ          		1.

2       503143          PLATE,BOT,LOWER RACK,LFT,40 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       			50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                				10.

12       503150          BLOCK,ROLLER MTG,SPRAYRACK,LWR          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            				10.

17       503149          BRACE,SPRAYRACK,LWR LFT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503138          BRACE,SPRAYRACK,TI,14.250"             			20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              				115.

26       003032          NUT,HEX,TI,1/4-20                     				115.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           			10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            			10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

SPRAYRACK ASSY,UP/RIGHT,POS607

503151          SPRAYRACK ASSY,UP/RIGHT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       503152          PLATE,TOP,UPPER RACK,RHT,30 NZ          		1.

2       503153          PLATE,BOT,UPPER RACK,RHT,30 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       			50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                				10.

12       503128          BLOCK,ROLLER MTG,OFFSET,POS607          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            				10.

17       503167          BRACE,SPRAYRACK,UPR RHT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503168          BRACE,SPRAYRACK,TI,11.875"             			20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              				104.

26       003032          NUT,HEX,TI,1/4-20                     				104.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           			10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            			10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

SPRAYRACK ASSY,LOW/RHGT,POS607

503169          SPRAYRACK ASSY,LOW/RHGT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       503170          PLATE,TOP,LOWER RACK,RHT,30 NZ          		1.

2       503171          PLATE,BOT,LOWER RACK,RHT,30 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       			50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                				10.

12       503150          BLOCK,ROLLER MTG,SPRAYRACK,LWR          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            				10.

17       503172          BRACE,SPRAYRACK,LWR RHT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503168          BRACE,SPRAYRACK,TI,11.875"             			20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              				104.

26       003032          NUT,HEX,TI,1/4-20                     				104.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           			10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            			10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

CNVR,ROLL,POS607,LR,N/OUT ROLL

#### 503129          CNVR,ROLL,POS607,LR,N/OUT ROLL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       500995          SIDERAIL,FRONT,33 ROD,POS07             			1.

2       500996          SIDERAIL,REAR,33 ROD,POS07              			1.

12       500932          STRUT,P-SIGMA,STD,TI  *                 			7.

13       500933          STRUT,P-SIGMA,L-R,TI                    			1.

14       501024          GEAR,MITER,13T,PVDF,.929,9P  *          			1.

15       501025          GEAR,MITER,13T,PVDF,1.500,9P *          			7.

16       501026          GEAR,MITER,13T,PVDF,1.662,9P           			24.

17       501027          GEAR,MITER,13T,BLIND,PVDF,9P *          			1.

18       503259          ROD,TIE,CNVR,TI,60                      				2.

19       003032          NUT,HEX,TI,1/4-20                       				8.

20       501358          SHAFT,LINE,TI,26.576                    				1.

21       501107          BRACE,SIDERAIL,POS07                    			2.

24       501359          SHAFT,LINE,TI,28.057                    				1.

25       003056          WASHER,FLAT,TI,1/4 NOM                  			4.

26       023353          PIN,GRAPHITE,1/4ODX1LG                  			4.

53       604176          BAFFLE,LWR,60,1-ROLL,PVC,EXT            			1.

54       604177          BAFFLE,UPPER,TI,60,POS607               			1.

55       502467          END PLATE,BAFFLE,POS                    			2.

56       503123          ROLL,60,2X2,SOLID,TPN/TI*               			1.

57       503130          ROLL,60,2X2,SOLID,LWR,TPN/TI            			8.

58       503281          ROD,60,2X2,LWR ONLY,GRAPHITE            		7.

59       106950          WHEEL,CNVR,TPN45D,2SX3/8,5HOLE        		108.

ROLL,60,2X2,SOLID,TPN/TI*

503123          ROLL,60,2X2,SOLID,TPN/TI*

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				2.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              				2.

7       137116          SHAFT ASSY,1PIN/SK,TI *                 				1.

8       501020          GEAR,SPUR,15T,EXT,PVDF       *          			1.

9       501041          GEAR,SPUR,15T,TABBED,PVDF    *          			1.

10       141546          CLIP,RETAINER,PPL                       				2.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

ROLL,60,2X2,SOLID,LWR,TPN/T

#### 503130          ROLL,60,2X2,SOLID,LWR,TPN/T

I

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	              QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				1.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              			1.

10       141546          CLIP,RETAINER,PPL                       				1.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

ROD,60,2X2,LWR ONLY,GRAPHITE

#### 503281          ROD,60,2X2,LWR ONLY,GRAPHITE

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       501007          ROD,30,2X2,LWR ONLY,FBRGL    *  			0.00000000

1       503282          ROD,GRAPHITE,3/8X65,D        *          			1.

2       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

3       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

PLUMBING,POS607

503105          PLUMBING,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       	503146          PLUMBING,POS607,LOWER RACK              		1.

2       	503165          PLUMBING,POS607,UPPER RACK              		1.

3       	503166          PLUMBING,POS607,DRAIN                   		1.

PLUMBING,POS607,LOWER RACK

503146          PLUMBING,POS607,LOWER RACK

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       501045          FILTER/CARAPACE,POS***,PVC              			1.

2       002401          ELBOW,90,PVC,1-1/2SOC,SCH80  S         			10.

3       025781          TEE,PVC,1-1/2SOC,SCH80       S          			3.

4       143841          VALVE,GATE,PVC,1-1/2SOC      S          			2.

5       153955          BUSHING,PVC,1-1/2MSX1FS      S          			4.

6       151193          CLAMP,TRIM-FIT,1",PVC/TI                				4.

7       002133          PIPE,PVC,1-1/2IN,SCH80       S      				12.000

8       153684          CLAMP,TRIM-FIT,1-1/2,PVC/TI             			2.

9       503144          HANDLE INSTL,POS60                      				2.

10       147259          CLAMP,PVC/TI,1-1/2,SCH80 FIT            			3.

11       003023          BOLT,HEX,TI,1/4-20X1-1/2                				4.

12       003024          BOLT,HEX,TI,1/4-20X1                    				4.

13       069565          PVC,1/2 X     1/2                   					24.000

14       002155          PIPE,PVC,1IN,SCH80           S      				10.000

15       503157          HOSE ASSY,MECHANICAL,PVC,POS60          		4.

16       002710          SCREW,RD,TI,1/4-20 X 1"                 				8.

PLUMBING,POS607,UPPER RACK

503165          PLUMBING,POS607,UPPER RACK

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       501045          FILTER/CARAPACE,POS***,PVC              			1.

2       002401          ELBOW,90,PVC,1-1/2SOC,SCH80  S         			10.

3       025781          TEE,PVC,1-1/2SOC,SCH80       S          			3.

4       143841          VALVE,GATE,PVC,1-1/2SOC      S          			2.

5       153955          BUSHING,PVC,1-1/2MSX1FS      S          			4.

6       151193          CLAMP,TRIM-FIT,1",PVC/TI                				4.

7       002133          PIPE,PVC,1-1/2IN,SCH80       S      				12.000

8       153684          CLAMP,TRIM-FIT,1-1/2,PVC/TI             			2.

9       503144          HANDLE INSTL,POS60                      				2.

10       147259          CLAMP,PVC/TI,1-1/2,SCH80 FIT            			3.

11       003023          BOLT,HEX,TI,1/4-20X1-1/2                				4.

12       003024          BOLT,HEX,TI,1/4-20X1                    				4.

13       069565          PVC,1/2 X     1/2                   					24.000

14       002155          PIPE,PVC,1IN,SCH80           S      				10.000

15       503157          HOSE ASSY,MECHANICAL,PVC,POS60          		4.

16       002710          SCREW,RD,TI,1/4-20 X 1"                 				8.

17       145520          SUPPORT,PLUMB,1-1/2,OS                  			2.

PLUMBING,POS607,DRAIN

503166          PLUMBING,POS607,DRAIN

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       018454          VALVE,BALL,PVC,1-1/2NPT,TU V S          			1.

0       106242          NIPPLE,PVC,1-1/2"(TXS)5"LG              			1.

PUMP SUBASSY,P-55J-C1

138498          PUMP SUBASSY,P-55J-C1

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       145430          PUMP SUBASSEMBLY(REF.DWG)              			0.0

0       147234          SPECIFICATION,PUMPS,CHEMCUT            		0.0

1       025261          MOTOR,AC,7-1/2HP(60HZ)5HP(50HZ          			1.

2       080073          COWL,MOTOR,LARGE,3,5,7-1/2 HP           			1.

3       037989          SCREW,SELF TAP,10-32X3/4                			1.

4       084594          COVER,HOLE,BLACK,1 INCH                 			1.

5       133714          MOUNT,MOTOR,RPP,P-25                    			1.

6       043122          BOLT,HEX,SST316,1/2-13X1-1/4            			4.

7       040328          WASHER,FLAT,TI,1/2"NOM                  			4.

8       043089          WASHER,LOCK,SST316,1/2"NOM              			4.

9       163463          SHAFT,PUMP,TI,1-1/2DIAX7/8-14*          			1.

11       105644          COLUMN ASSY,PUMP,PPL (SH)               			1.

12       105492          SLEEVE,PUMP COLUMN,TFE,P-24/25          		1.

13       031925          SEAL,O-RING,2-225,EPDM,BLACK            			1.

14       145908          PLATE,PUMP,OS,0DEG,1-1/2"               			1.

15       075961          KEY,"L",TI,1/4X1/4X1"LG                 				1.

16       104608          COLLAR,SHAFT,SST303,1-1/2ID             			1.

17       029171          COLLAR,MODIFIED                         				1.

18       170012          SEAL,O-RING,2-365,EPDM,BLACK            			1.

19       126509          IMPELLER,TI,7/8-14,5.75X5.75,T          			1.

20       135380          BOLT,HEX,TI,1/4-20X 3/4                 				1.

21       015811          WASHER,FLAT,TI,9/32IDX1-1/4OD           			1.

22       149058          SEAL,O-RING,2-360,EPDM,BLACK            			1.

23       003069          LABEL,MYLAR,(PUMP)                      				1.

24       014133          LABEL,WARNING,(PUMP ROTATION)           		1.

25       015392          LABEL,IDENT,(ROTATION,R-L)              			1.

26       145721          LABEL,"CAUTION HOT"                     				1.

Pump Specifications

COOLING,POS607

COOLING,POS607

503099          COOLING,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       176034          COIL,COOLING,TI,DPEM                    				1.

2       141853          GASKET,COOLING COIL,1.5 X 2             			2.

3       003056          WASHER,FLAT,TI,1/4 NOM                  			8.

4       003032          NUT,HEX,TI,1/4-20                       				8.

5       014819          CLAMP,COIL,PVC,3/4IN                    				3.

6       023837          UNION,PVC,3/4NPT,SCH80,EPDM  S          			1.

7       028992          ELBOW,90,PVC,3/4SOC,SCH80    S          			1.

8       011905          VALVE,SOL,PAINT,BRS,3/4NPT,N/C          			1.

9       002165          PIPE,PVC,3/4IN,SCH80         S       				7.750

10       150674          CLAMP,TRIM-FIT,3/4",PVC                 				1.

11       141381          COOLING COIL,DRAIN SUPPORT              			1.

12       069565          PVC,1/2 X     1/2                    					2.500

13       002380          ELBOW,90,PVC,3/4NPT,SCH80    S          			1.

14       080302          UNION,PVC,3/4SOC,SCH80,VITON S          			2.

15       080063          ELBOW,45,PVC,3/4SOC,SCH40               			2.

GAUGE INSTL,POS607,0-60 (40)

503113          GAUGE INSTL,POS607,0-60 (40)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       103650          GAUGE,PRESS,0-60/4BAR,W/ISOL           			 40.

0       003833          CONNECTOR,MALE,PP,1/4TX1/4NPT          			 40.

0       017905          INSERT,PP,1/4OD TUBE                   				 80.

0       011974          TUBING,PE,1/4"ODX.040 W            			          450.000

0       016108          ELBOW,MALE,PP,1/4TX1/4NPT              			 40.

0       137355          SHEET,PVC,EUROGRY,1/2X51X122IN       			   8.000

0       137353          SHEET,PVC,EUROGRY,1/8X48X96IN        			   8.000

0       002712          SCREW,RD,TI,1/4-20 X 1/2               				  40.

0       025980          RETAINER,TUBE,PVC,MOLDED              			120.

14       501791          INSERT,MALE,1/8"ID,PPL                 				  40.

15       501792          INSERT,FEMALE,1/8"ID,PPL               			  40.

0       144178          THUMBSCREW,TI,1/4-20X1-1/2,CAP          			    8.

0       023353          PIN,GRAPHITE,1/4ODX1LG                 			  16.

0       133930          INSERT,THD,TI,1/4-20X.472               				    8.

0       002709          SCREW,RD,TI,1/4-20 X 1-1/2"            				  16.

21       503216          COVER ASSY,20 PRESSURE GAUGE            		    2.

22       503218          COUPLING ASSY,10 HOSE                   			    4.

23       012803          GASKET,RD,TY,7/32"IDX1"ODX1/4          			  20.

GAUGE INSTL,PRESS,PEM,(2)

170088          GAUGE INSTL,PRESS,PEM,(2)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		    QUANTITY

1       103650          GAUGE,PRESS,0-60/4BAR,W/ISOL         			2.000

2       170404          COVER ASSY,PG,PVC,(2)                				1.000

3       002711          SCREW,RD,TI,1/4-20 X 3/4"            				2.000

4       011974          TUBING,PE,1/4"ODX.040 W             		                    12.000

5       025980          RETAINER,TUBE,PVC,MOLDED             			2.000

6       016108          ELBOW,MALE,PP,1/4TX1/4NPT            			4.000

7       017905          INSERT,PP,1/4OD TUBE                 				4.000

8       069565          PVC,1/2 X     1/2                    					2.625

8       069565          PVC,1/2 X     1/2                    					2.625

NOZZLE INSTL,EDGE ETCH

503223          NOZZLE INSTL,EDGE ETCH

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        QUANTITY

1       103650          GAUGE,PRESS,0-60/4BAR,W/ISOL            2.

2       156718          VALVE,NEEDLE,PVC,1/4NPT                 2.

3       106935          TEE,MALE RUN,PPL,1/4NPTX1/4T            3.

4       503225          CLAMP,MTG,GAUGE/VALVE                   1.

5       503224          BLOCK,MTG,NOZZLE                        2.

6       002710          SCREW,RD,TI,1/4-20 X 1"                 2.

7       149941          NOZZLE,SCRD,PVDF,.75GPM,80DEG           2.

8       016108          ELBOW,MALE,PP,1/4TX1/4NPT               2.

9       003833          CONNECTOR,MALE,PP,1/4TX1/4NPT           4.

10       011974          TUBING,PE,1/4"ODX.040 W             25.000

11       012803          GASKET,RD,TY,7/32"IDX1"ODX1/4           4.

12       069565          PVC,1/2 X     1/2                    7.000

13       017905          INSERT,PP,1/4OD TUBE                   12.

14       002709          SCREW,RD,TI,1/4-20 X 1-1/2"             1.

HEAT,6X6KW,440-480V,TI,M BLD

503126          HEAT,6X6KW,440-480V,TI,M BLD

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

0       133384          HEATER INSTL,TOS08(REF DWG)             			0.

1       128351          HEATER,IMM,6KW,480V,STUD,TI             			6.

2       149937          INSERT,TPE,.440,CORD,MOLDED            			12.

3       120526          CONNECTOR,BOX,HEATER,PP,1/2            			12.

4       161614          SWITCH,LIQUID LEVEL ACTUATOR            			1.

5       117113          CONNECTOR ASSY,BOX,PP,1/2PIPE           		1.

6       149368          SEAL,O-RING,2-211,EPDM                  			1.

7       117111          CONNECTOR,BOX,PROBE,PP,1/2              			2.

8       046328          INSERT,TPE,1/2,CORD,MOLDED              			2.

9       117126          CONNECTOR,BOX,BLANK,PP                  			2.

10       046330          INSERT,TPE,7/32,CORD,MOLDED             			2.

0       164422          PROBE,T-S/OT,132 F,14 IN,POLY        			1.000

13       133651          BOOT,HEATER TERMINAL COVER             		           12.

JCT BOX INSTL,AIRGAP,POS06

501676          JCT BOX INSTL,AIRGAP,POS06

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       	501677          BOX ASSY,AIRGAP,POS06/08               		1.0

RECIRC PUMP,(2)POS,3HP,W/FEEDS

502817          RECIRC PUMP,(2)POS,3HP,W/FEEDS

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       147075          PUMP,CENT,MD,3HPCE,KC22,NPT,RP          		1.

2       135409          STAND,RECIRC,PUMP,OS,3 1/8"             			1.

3       003010          BOLT,HEX,TI,3/8-16X1                    				4.

4       009314          WASHER,FLAT,TI,3/8"NOM                  			4.

5       135302          ADAPTER,PLUMBING,THRU WALL,2"           		2.

6       018455          VALVE,BALL,PVC,2NPT,TU                  			2.

7       118537          UNION,PVC,2SOC,SCH80,EPDM               			1.

8       015637          ELBOW,90,PVC,2SOC,SCH80      S          			4.

9       034708          ELBOW,45,PVC,2SOC,SCH80                 			1.

10       153825          CLAMP,TRIM-FIT,2",PVC                   				2.

12       006724          PIPE,PVC,2IN,SCH80                  				10.000

13       025778          TEE,PVC,2SOC,SCH80                      				1.

14       028990          BUSHING,PVC,2MSX3/4FS                   			1.

15       002165          PIPE,PVC,3/4IN,SCH80         S          				1.

16       028991          TEE,PVC,3/4SOC,SCH80         S          			1.

17       118535          UNION,PVC,3/4SOC,SCH80,EPDM  S          		2.

18       142520          BUSHING,PVC,3/4MSX1/4FPT                			2.

19       016108          ELBOW,MALE,PP,1/4TX1/4NPT               			2.

20       017905          INSERT,PP,1/4OD TUBE                    				2.

21       011974          TUBING,PE,1/4"ODX.040 W             				20.000

22       023949          ELBOW,90,PVC,2NPTX2SOC                  			1.

INTERCONNECT,SUMP,POS (4-EA)

503137          INTERCONNECT,SUMP,POS (4-EA)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

0       T-2984          TOOL,WRENCH SPANNER,2 PIECES            		0.

1       142665          SUMP INTERCONNECT,SIGMA,(2)             			2.

142665          SUMP INTERCONNECT,SIGMA,(2)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       084601          ADAPTER ASSY,SUMP INTERCONNECT          		2.

2       143114          RING,SUMP INTERCONNECT,OS               			2.

3       022672          SEAL,O-RING,2-362,EPDM,BLACK            			2.

SUPPLY,PVC SOL,1/2",POS

503156          SUPPLY,PVC SOL,1/2",POS

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       148694          COUPLING,PVC,1/2SOC,MOD                			1.0

2       002176          PIPE,PVC,1/2IN,SCH80                   				1.0

3       118109          ELBOW,90,PVC,1/2SOC,SCH80    S         			1.0

4       118534          UNION,PVC,1/2SOC,SCH80       S         			1.0

5       163838          VALVE,SOL,PVC,1/2NPT,N/C,2WAY*         			1.0

POS607        Ferric Chloride Etch

POS607M-1       POS607,MEC PARTS,NO/AIR SEAL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	 QUANTITY

0       503094          POS607 MOD ASSY,NO/AIR SEAL             		1.

0       503095          MODULE INT,POS607,TI,N/AIRSEAL          		1.

POS607 MOD ASSY,NO/AIR SEAL

#### 503094          POS607 MOD ASSY,NO/AIR SEAL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       503056          CHAMBER,POS607,NO/AIR SEAL              			1.

0       503105          PLUMBING,POS607                         				1.

3       503106          SPRAYRACK INSTL,POS607,(40-PG)          		1.

0       503097          DRIVE INSTL,OSCILLATION,POS607          		2.

0       503099          COOLING,POS607                          				2.

0       503107          JCT BOX INSTL,AIRGAP,POS607             			1.

0       131577          COVER INSTL,FRONT,TOS*06                			1.

0       144090          COVER INSTL,OSC,OS                      			2.

0       501748          COVER,SUMP ACCESS/SAMPLE,POS            		1.

0       501749          COVER,SUMP ACCESS,POS                   			2.

0       600404          PLUG,PORT,PVC,XLI                       				1.

0       170088          GAUGE INSTL,PRESS,PEM,(2)               			2.

136179          LABELS,SIGMA,TOS,STD                    			1

502809          BAG,FILTER,80MIC,PLYESTR,(2)            			1

502817          RECIRC PUMP,(2)POS,3HP,W/FEEDS          		1

503113          GAUGE INSTL,POS607,0-60 (40)            			1

503126          HEAT,6X6KW,440-480V,TI,M BLD            			1

503129          CNVR,ROLL,POS607,LR,N/OUT ROLL          		1

503135          PUMP,2X7.5,P-55CJ-CI,TI IMPELL          			1

503136          NOZZLE,1.5GPM,073711,SCRD(140)          		1

503137          INTERCONNECT,SUMP,POS (4-EA)            		1

503156          SUPPLY,PVC SOL,1/2",POS                 			1

503223	NOZZLE INSTL,EDGE ETCH				1

MODULE INT,POS607,TI,N/AIRSEAL

503095          MODULE INT,POS607,TI,N/AIRSEAL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	 QUANTITY

0       503108          DOOR INSTL,FRONT,POS*07,N/AIRS          		1.

0       503192          DOOR INSTL,REAR,POS*07,N/AIRS           		1.

0       503109          LID INSTL,TOP POS607,N/AIRSEAL          		1.

0       503096          GASKET,CNVR,P-SIGMA,60,TPE55A           		1.

0       503104          ANGLE,OPENING,P-SIGMA,60,TI             		4.

0       141115          BOLT,HEX,TI,1/4-20X1-1/4               		         58.

0       003032          NUT,HEX,TI,1/4-20                      		         58.

0       142534          PIN,GRAPHITE,1/4ODX3/4LG                		2.

0       143787          LABEL,UPPER,INPUT                       			1.

0       143788          LABEL,UPPER,OUTPUT                      		1.

0       143789          LABEL,LOWER,INPUT                       			1.

0       143790          LABEL,LOWER,OUTPUT                      		1.

0       049817          ADAPTER,VENT,PVC,MOLDED,4"              		2.

0       501035          PLATE,CONN,FRONT,P-SIGM,TI              		2.

0       501036          PLATE,CONN,REAR,P-SIGMA,TI              		2.

DRIVE INSTL,OSCILLATION,POS607	503097 No Drawing

503097          DRIVE INSTL,OSCILLATION,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       144240          MOTOR,GEAR,DC,1/4HP,68RPM,RITE          		1.

2       144630          CLAMP ASSY,MOTOR,OSC,PVC,TOS            		1.

3       003024          BOLT,HEX,TI,1/4-20X1                    				4.

4       003056          WASHER,FLAT,TI,1/4 NOM                  			4.

5       501596          BUSHING,UPPER,OSC,POS                   			1.

6       069565          PVC,1/2 X     1/2                   				         12.000

7       501718          BEARING,LOWER,OSC,UHMW                  			1.

8       031243          PIN,TI,.125ODX1LG                       				2.

9       144243          COUPLING,DRIVE,OSC,OS                   			1.

10       148916          KEY,SQ TI,3/16X1-1/2LG                  				1.

11       029163          SETSCREW,TI,1/4-20X1/4",CUP PT          			2.

12       503085          PLATE,SEAL,OSCILLATION,POS,MOD          		1.

12       503092          BRACKET,SHAFT,OSC,LWR,POS607       			0.00000

13       159857          SEAL,O-RING,2-241,EPDM,BLACK            			2.

14       002711          SCREW,RD,TI,1/4-20 X 3/4"               				4.

15       082298          SEAL,V-RING,CVU-1220,PTFE               			1.

16       148952          CAP,BEARING,PVC                         				1.

17       502731          SHAFT ASSY,OSC,POS                      			1.

18       501080          CAM,OSCILLATION,POS                     			2.

19       003008          BOLT,HEX,TI,3/8-16X1-1/2                				2.

20       003012          NUT,HEX,TI,3/8-16                       				2.

SPRAYRACK INSTL,POS607,(40-PG)

Lower Rack Shown

503106          SPRAYRACK INSTL,POS607,(40-PG)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

0       503114          SPRAYRACK ASSY,UP/LEFT,POS607           		1.

0       503141          SPRAYRACK ASSY,LOW/LEFT,POS607          		1.

0       503151          SPRAYRACK ASSY,UP/RIGHT,POS607          		1.

0       503169          SPRAYRACK ASSY,LOW/RHGT,POS607          		1.

5       503120          PIN,RACK SUPPORT ROLLER                 			8.

6       503119          ROLLER,RACK SUPPORT                    		         16.

7       130994          WASHER,HDPE,.380IDX1ODX1/16THK         	         16.

8       503121          COLLAR,LOCK,RACK ROLLER                 			8.

SPRAYRACK ASSY,UP/LEFT,POS607

503114          SPRAYRACK ASSY,UP/LEFT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

1       503115          PLATE,TOP,UPPER RACK,LFT,40 NZ          		1.

2       503116          PLATE,BOT,UPPER RACK,LFT,40 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       		         50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                			         10.

12       503128          BLOCK,ROLLER MTG,OFFSET,POS607          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            			         10.

17       503132          BRACE,SPRAYRACK,UPR LFT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503138          BRACE,SPRAYRACK,TI,14.250"             		         20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              			       115.

26       003032          NUT,HEX,TI,1/4-20                     			       115.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           		         10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            		         10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

SPRAYRACK ASSY,LOW/LEFT,POS607

503141          SPRAYRACK ASSY,LOW/LEFT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

1       503142          PLATE,TOP,LOWER RACK,LFT,40 NZ          		1.

2       503143          PLATE,BOT,LOWER RACK,LFT,40 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       			50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                				10.

12       503150          BLOCK,ROLLER MTG,SPRAYRACK,LWR          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            				10.

17       503149          BRACE,SPRAYRACK,LWR LFT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503138          BRACE,SPRAYRACK,TI,14.250"             			20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              				115.

26       003032          NUT,HEX,TI,1/4-20                     				115.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           			10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            			10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

SPRAYRACK ASSY,UP/RIGHT,POS607

503151          SPRAYRACK ASSY,UP/RIGHT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       503152          PLATE,TOP,UPPER RACK,RHT,30 NZ          		1.

2       503153          PLATE,BOT,UPPER RACK,RHT,30 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       			50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                				10.

12       503128          BLOCK,ROLLER MTG,OFFSET,POS607          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            				10.

17       503167          BRACE,SPRAYRACK,UPR RHT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503168          BRACE,SPRAYRACK,TI,11.875"             			20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              				104.

26       003032          NUT,HEX,TI,1/4-20                     				104.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           			10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            			10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

SPRAYRACK ASSY,LOW/RHGT,POS607

503169          SPRAYRACK ASSY,LOW/RHGT,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       503170          PLATE,TOP,LOWER RACK,RHT,30 NZ          		1.

2       503171          PLATE,BOT,LOWER RACK,RHT,30 NZ          		1.

0       700404          SEAL,O-RING,EPDM,1/8 X RANDOM       			50.000

9       501073          ROLLER GUIDE,SPRAY RACK                 			8.

10       502552          PIN,ROLLER,SPRAYRACK                    			8.

11       002710          SCREW,RD,TI,1/4-20 X 1"                				10.

12       503150          BLOCK,ROLLER MTG,SPRAYRACK,LWR          		4.

13       003029          SCREW,RD,TI,1/4-20 X 1-1/4"            				10.

17       503172          BRACE,SPRAYRACK,LWR RHT,65.25"          		1.

19       503134          BRACE,SPRAYRACK,TI,65.250"              			1.

23       503168          BRACE,SPRAYRACK,TI,11.875"             			20.

25       009356          BOLT,HEX,TI,1/4-20X1-3/4              				104.

26       003032          NUT,HEX,TI,1/4-20                     				104.

28       501098          ADAPTER,ADJUST PLUG,1-14 UNF           			10.

31       501097          PLUG,ADJUSTING,PPL,1-14 UNF            			10.

37       145556          SCREW,FLAT,TI,10-24 X 3/4"              			2.

39       503139          CAM LOCK INSTL,POS60*                   			1.

41       142534          PIN,GRAPHITE,1/4ODX3/4LG                			2.

42       003056          WASHER,FLAT,TI,1/4 NOM                  			6.

CNVR,ROLL,POS607,LR,N/OUT ROLL

#### 503129          CNVR,ROLL,POS607,LR,N/OUT ROLL

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       500995          SIDERAIL,FRONT,33 ROD,POS07             			1.

2       500996          SIDERAIL,REAR,33 ROD,POS07              			1.

12       500932          STRUT,P-SIGMA,STD,TI  *                 			7.

13       500933          STRUT,P-SIGMA,L-R,TI                    			1.

14       501024          GEAR,MITER,13T,PVDF,.929,9P  *          			1.

15       501025          GEAR,MITER,13T,PVDF,1.500,9P *          			7.

16       501026          GEAR,MITER,13T,PVDF,1.662,9P           			24.

17       501027          GEAR,MITER,13T,BLIND,PVDF,9P *          			1.

18       503259          ROD,TIE,CNVR,TI,60                      				2.

19       003032          NUT,HEX,TI,1/4-20                       				8.

20       501358          SHAFT,LINE,TI,26.576                    				1.

21       501107          BRACE,SIDERAIL,POS07                    			2.

24       501359          SHAFT,LINE,TI,28.057                    				1.

25       003056          WASHER,FLAT,TI,1/4 NOM                  			4.

26       023353          PIN,GRAPHITE,1/4ODX1LG                  			4.

53       604176          BAFFLE,LWR,60,1-ROLL,PVC,EXT            			1.

54       604177          BAFFLE,UPPER,TI,60,POS607               			1.

55       502467          END PLATE,BAFFLE,POS                    			2.

56       503123          ROLL,60,2X2,SOLID,TPN/TI*               			1.

57       503130          ROLL,60,2X2,SOLID,LWR,TPN/TI            			8.

58       503281          ROD,60,2X2,LWR ONLY,GRAPHITE            		7.

59       106950          WHEEL,CNVR,TPN45D,2SX3/8,5HOLE        		108.

ROLL,60,2X2,SOLID,TPN/TI*

503123          ROLL,60,2X2,SOLID,TPN/TI*

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				2.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              				2.

7       137116          SHAFT ASSY,1PIN/SK,TI *                 				1.

8       501020          GEAR,SPUR,15T,EXT,PVDF       *          			1.

9       501041          GEAR,SPUR,15T,TABBED,PVDF    *          			1.

10       141546          CLIP,RETAINER,PPL                       				2.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

ROLL,60,2X2,SOLID,LWR,TPN/T

#### 503130          ROLL,60,2X2,SOLID,LWR,TPN/T

I

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	              QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				1.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              			1.

10       141546          CLIP,RETAINER,PPL                       				1.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

ROD,60,2X2,LWR ONLY,GRAPHITE

#### 503281          ROD,60,2X2,LWR ONLY,GRAPHITE

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       501007          ROD,30,2X2,LWR ONLY,FBRGL    *  			0.00000000

1       503282          ROD,GRAPHITE,3/8X65,D        *          			1.

2       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

3       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

PLUMBING,POS607

503105          PLUMBING,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       	503146          PLUMBING,POS607,LOWER RACK              		1.

2       	503165          PLUMBING,POS607,UPPER RACK              		1.

3       	503166          PLUMBING,POS607,DRAIN                   		1.

PLUMBING,POS607,LOWER RACK

503146          PLUMBING,POS607,LOWER RACK

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       501045          FILTER/CARAPACE,POS***,PVC              			1.

2       002401          ELBOW,90,PVC,1-1/2SOC,SCH80  S         			10.

3       025781          TEE,PVC,1-1/2SOC,SCH80       S          			3.

4       143841          VALVE,GATE,PVC,1-1/2SOC      S          			2.

5       153955          BUSHING,PVC,1-1/2MSX1FS      S          			4.

6       151193          CLAMP,TRIM-FIT,1",PVC/TI                				4.

7       002133          PIPE,PVC,1-1/2IN,SCH80       S      				12.000

8       153684          CLAMP,TRIM-FIT,1-1/2,PVC/TI             			2.

9       503144          HANDLE INSTL,POS60                      				2.

10       147259          CLAMP,PVC/TI,1-1/2,SCH80 FIT            			3.

11       003023          BOLT,HEX,TI,1/4-20X1-1/2                				4.

12       003024          BOLT,HEX,TI,1/4-20X1                    				4.

13       069565          PVC,1/2 X     1/2                   					24.000

14       002155          PIPE,PVC,1IN,SCH80           S      				10.000

15       503157          HOSE ASSY,MECHANICAL,PVC,POS60          		4.

16       002710          SCREW,RD,TI,1/4-20 X 1"                 				8.

PLUMBING,POS607,UPPER RACK

503165          PLUMBING,POS607,UPPER RACK

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       501045          FILTER/CARAPACE,POS***,PVC              			1.

2       002401          ELBOW,90,PVC,1-1/2SOC,SCH80  S         			10.

3       025781          TEE,PVC,1-1/2SOC,SCH80       S          			3.

4       143841          VALVE,GATE,PVC,1-1/2SOC      S          			2.

5       153955          BUSHING,PVC,1-1/2MSX1FS      S          			4.

6       151193          CLAMP,TRIM-FIT,1",PVC/TI                				4.

7       002133          PIPE,PVC,1-1/2IN,SCH80       S      				12.000

8       153684          CLAMP,TRIM-FIT,1-1/2,PVC/TI             			2.

9       503144          HANDLE INSTL,POS60                      				2.

10       147259          CLAMP,PVC/TI,1-1/2,SCH80 FIT            			3.

11       003023          BOLT,HEX,TI,1/4-20X1-1/2                				4.

12       003024          BOLT,HEX,TI,1/4-20X1                    				4.

13       069565          PVC,1/2 X     1/2                   					24.000

14       002155          PIPE,PVC,1IN,SCH80           S      				10.000

15       503157          HOSE ASSY,MECHANICAL,PVC,POS60          		4.

16       002710          SCREW,RD,TI,1/4-20 X 1"                 				8.

17       145520          SUPPORT,PLUMB,1-1/2,OS                  			2.

PLUMBING,POS607,DRAIN

503166          PLUMBING,POS607,DRAIN

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       018454          VALVE,BALL,PVC,1-1/2NPT,TU V S          			1.

0       106242          NIPPLE,PVC,1-1/2"(TXS)5"LG              			1.

PUMP SUBASSY,P-55J-C1

138498          PUMP SUBASSY,P-55J-C1

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       145430          PUMP SUBASSEMBLY(REF.DWG)              			0.0

0       147234          SPECIFICATION,PUMPS,CHEMCUT            		0.0

1       025261          MOTOR,AC,7-1/2HP(60HZ)5HP(50HZ          			1.

2       080073          COWL,MOTOR,LARGE,3,5,7-1/2 HP           			1.

3       037989          SCREW,SELF TAP,10-32X3/4                			1.

4       084594          COVER,HOLE,BLACK,1 INCH                 			1.

5       133714          MOUNT,MOTOR,RPP,P-25                    			1.

6       043122          BOLT,HEX,SST316,1/2-13X1-1/4            			4.

7       040328          WASHER,FLAT,TI,1/2"NOM                  			4.

8       043089          WASHER,LOCK,SST316,1/2"NOM              			4.

9       163463          SHAFT,PUMP,TI,1-1/2DIAX7/8-14*          			1.

11       105644          COLUMN ASSY,PUMP,PPL (SH)               			1.

12       105492          SLEEVE,PUMP COLUMN,TFE,P-24/25          		1.

13       031925          SEAL,O-RING,2-225,EPDM,BLACK            			1.

14       145908          PLATE,PUMP,OS,0DEG,1-1/2"               			1.

15       075961          KEY,"L",TI,1/4X1/4X1"LG                 				1.

16       104608          COLLAR,SHAFT,SST303,1-1/2ID             			1.

17       029171          COLLAR,MODIFIED                         				1.

18       170012          SEAL,O-RING,2-365,EPDM,BLACK            			1.

19       126509          IMPELLER,TI,7/8-14,5.75X5.75,T          			1.

20       135380          BOLT,HEX,TI,1/4-20X 3/4                 				1.

21       015811          WASHER,FLAT,TI,9/32IDX1-1/4OD           			1.

22       149058          SEAL,O-RING,2-360,EPDM,BLACK            			1.

23       003069          LABEL,MYLAR,(PUMP)                      				1.

24       014133          LABEL,WARNING,(PUMP ROTATION)           		1.

25       015392          LABEL,IDENT,(ROTATION,R-L)              			1.

26       145721          LABEL,"CAUTION HOT"                     				1.

Pump Specifications

COOLING,POS607

COOLING,POS607

503099          COOLING,POS607

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       176034          COIL,COOLING,TI,DPEM                    				1.

2       141853          GASKET,COOLING COIL,1.5 X 2             			2.

3       003056          WASHER,FLAT,TI,1/4 NOM                  			8.

4       003032          NUT,HEX,TI,1/4-20                       				8.

5       014819          CLAMP,COIL,PVC,3/4IN                    				3.

6       023837          UNION,PVC,3/4NPT,SCH80,EPDM  S          			1.

7       028992          ELBOW,90,PVC,3/4SOC,SCH80    S          			1.

8       011905          VALVE,SOL,PAINT,BRS,3/4NPT,N/C          			1.

9       002165          PIPE,PVC,3/4IN,SCH80         S       				7.750

10       150674          CLAMP,TRIM-FIT,3/4",PVC                 				1.

11       141381          COOLING COIL,DRAIN SUPPORT              			1.

12       069565          PVC,1/2 X     1/2                    					2.500

13       002380          ELBOW,90,PVC,3/4NPT,SCH80    S          			1.

14       080302          UNION,PVC,3/4SOC,SCH80,VITON S          			2.

15       080063          ELBOW,45,PVC,3/4SOC,SCH40               			2.

GAUGE INSTL,POS607,0-60 (40)

503113          GAUGE INSTL,POS607,0-60 (40)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       103650          GAUGE,PRESS,0-60/4BAR,W/ISOL           			 40.

0       003833          CONNECTOR,MALE,PP,1/4TX1/4NPT          			 40.

0       017905          INSERT,PP,1/4OD TUBE                   				 80.

0       011974          TUBING,PE,1/4"ODX.040 W            			          450.000

0       016108          ELBOW,MALE,PP,1/4TX1/4NPT              			 40.

0       137355          SHEET,PVC,EUROGRY,1/2X51X122IN       			   8.000

0       137353          SHEET,PVC,EUROGRY,1/8X48X96IN        			   8.000

0       002712          SCREW,RD,TI,1/4-20 X 1/2               				  40.

0       025980          RETAINER,TUBE,PVC,MOLDED              			120.

14       501791          INSERT,MALE,1/8"ID,PPL                 				  40.

15       501792          INSERT,FEMALE,1/8"ID,PPL               			  40.

0       144178          THUMBSCREW,TI,1/4-20X1-1/2,CAP          			    8.

0       023353          PIN,GRAPHITE,1/4ODX1LG                 			  16.

0       133930          INSERT,THD,TI,1/4-20X.472               				    8.

0       002709          SCREW,RD,TI,1/4-20 X 1-1/2"            				  16.

21       503216          COVER ASSY,20 PRESSURE GAUGE            		    2.

22       503218          COUPLING ASSY,10 HOSE                   			    4.

23       012803          GASKET,RD,TY,7/32"IDX1"ODX1/4          			  20.

GAUGE INSTL,PRESS,PEM,(2)

170088          GAUGE INSTL,PRESS,PEM,(2)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		    QUANTITY

1       103650          GAUGE,PRESS,0-60/4BAR,W/ISOL         			2.000

2       170404          COVER ASSY,PG,PVC,(2)                				1.000

3       002711          SCREW,RD,TI,1/4-20 X 3/4"            				2.000

4       011974          TUBING,PE,1/4"ODX.040 W             		                    12.000

5       025980          RETAINER,TUBE,PVC,MOLDED             			2.000

6       016108          ELBOW,MALE,PP,1/4TX1/4NPT            			4.000

7       017905          INSERT,PP,1/4OD TUBE                 				4.000

8       069565          PVC,1/2 X     1/2                    					2.625

8       069565          PVC,1/2 X     1/2                    					2.625

NOZZLE INSTL,EDGE ETCH

503223          NOZZLE INSTL,EDGE ETCH

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        QUANTITY

1       103650          GAUGE,PRESS,0-60/4BAR,W/ISOL            2.

2       156718          VALVE,NEEDLE,PVC,1/4NPT                 2.

3       106935          TEE,MALE RUN,PPL,1/4NPTX1/4T            3.

4       503225          CLAMP,MTG,GAUGE/VALVE                   1.

5       503224          BLOCK,MTG,NOZZLE                        2.

6       002710          SCREW,RD,TI,1/4-20 X 1"                 2.

7       149941          NOZZLE,SCRD,PVDF,.75GPM,80DEG           2.

8       016108          ELBOW,MALE,PP,1/4TX1/4NPT               2.

9       003833          CONNECTOR,MALE,PP,1/4TX1/4NPT           4.

10       011974          TUBING,PE,1/4"ODX.040 W             25.000

11       012803          GASKET,RD,TY,7/32"IDX1"ODX1/4           4.

12       069565          PVC,1/2 X     1/2                    7.000

13       017905          INSERT,PP,1/4OD TUBE                   12.

14       002709          SCREW,RD,TI,1/4-20 X 1-1/2"             1.

HEAT,6X6KW,440-480V,TI,M BLD

503126          HEAT,6X6KW,440-480V,TI,M BLD

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

0       133384          HEATER INSTL,TOS08(REF DWG)             			0.

1       128351          HEATER,IMM,6KW,480V,STUD,TI             			6.

2       149937          INSERT,TPE,.440,CORD,MOLDED            			12.

3       120526          CONNECTOR,BOX,HEATER,PP,1/2            			12.

4       161614          SWITCH,LIQUID LEVEL ACTUATOR            			1.

5       117113          CONNECTOR ASSY,BOX,PP,1/2PIPE           		1.

6       149368          SEAL,O-RING,2-211,EPDM                  			1.

7       117111          CONNECTOR,BOX,PROBE,PP,1/2              			2.

8       046328          INSERT,TPE,1/2,CORD,MOLDED              			2.

9       117126          CONNECTOR,BOX,BLANK,PP                  			2.

10       046330          INSERT,TPE,7/32,CORD,MOLDED             			2.

0       164422          PROBE,T-S/OT,132 F,14 IN,POLY        			1.000

13       133651          BOOT,HEATER TERMINAL COVER             		           12.

JCT BOX INSTL,AIRGAP,POS06

501676          JCT BOX INSTL,AIRGAP,POS06

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

0       	501677          BOX ASSY,AIRGAP,POS06/08               		1.0

RECIRC PUMP,(2)POS,3HP,W/FEEDS

502817          RECIRC PUMP,(2)POS,3HP,W/FEEDS

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       147075          PUMP,CENT,MD,3HPCE,KC22,NPT,RP          		1.

2       135409          STAND,RECIRC,PUMP,OS,3 1/8"             			1.

3       003010          BOLT,HEX,TI,3/8-16X1                    				4.

4       009314          WASHER,FLAT,TI,3/8"NOM                  			4.

5       135302          ADAPTER,PLUMBING,THRU WALL,2"           		2.

6       018455          VALVE,BALL,PVC,2NPT,TU                  			2.

7       118537          UNION,PVC,2SOC,SCH80,EPDM               			1.

8       015637          ELBOW,90,PVC,2SOC,SCH80      S          			4.

9       034708          ELBOW,45,PVC,2SOC,SCH80                 			1.

10       153825          CLAMP,TRIM-FIT,2",PVC                   				2.

12       006724          PIPE,PVC,2IN,SCH80                  				10.000

13       025778          TEE,PVC,2SOC,SCH80                      				1.

14       028990          BUSHING,PVC,2MSX3/4FS                   			1.

15       002165          PIPE,PVC,3/4IN,SCH80         S          				1.

16       028991          TEE,PVC,3/4SOC,SCH80         S          			1.

17       118535          UNION,PVC,3/4SOC,SCH80,EPDM  S          		2.

18       142520          BUSHING,PVC,3/4MSX1/4FPT                			2.

19       016108          ELBOW,MALE,PP,1/4TX1/4NPT               			2.

20       017905          INSERT,PP,1/4OD TUBE                    				2.

21       011974          TUBING,PE,1/4"ODX.040 W             				20.000

22       023949          ELBOW,90,PVC,2NPTX2SOC                  			1.

INTERCONNECT,SUMP,POS (4-EA)

503137          INTERCONNECT,SUMP,POS (4-EA)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		QUANTITY

0       T-2984          TOOL,WRENCH SPANNER,2 PIECES            		0.

1       142665          SUMP INTERCONNECT,SIGMA,(2)             			2.

142665          SUMP INTERCONNECT,SIGMA,(2)

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       084601          ADAPTER ASSY,SUMP INTERCONNECT          		2.

2       143114          RING,SUMP INTERCONNECT,OS               			2.

3       022672          SEAL,O-RING,2-362,EPDM,BLACK            			2.

SUPPLY,PVC SOL,1/2",POS

503156          SUPPLY,PVC SOL,1/2",POS

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       148694          COUPLING,PVC,1/2SOC,MOD                			1.0

2       002176          PIPE,PVC,1/2IN,SCH80                   				1.0

3       118109          ELBOW,90,PVC,1/2SOC,SCH80    S         			1.0

4       118534          UNION,PVC,1/2SOC,SCH80       S         			1.0

5       163838          VALVE,SOL,PVC,1/2NPT,N/C,2WAY*         			1.0

PIA634        Conveyor

PIA634M         PIA634,MEC PARTS,P-SIGMA

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

0       503183          CHAMBER,PIA634                          				1.

3       750034          COVER,SIDERAIL,REAR,PIA334              			1.

4       750035          COVER,SIDERAIL,FRONT,PIA334             			1.

5       503159          PLENUM,PRE634                           				1.

5       503188          PLENUM,PLE634                           				1.

6       131644          COVER INSTL,FRONT,33.882                			1.

7       503158          MOD.INT.PARTS,P-SIGMA,60,TI             			1.

136186          LABELS,SIGMA,TIA,STD                    			1

503127          CNVR,ROLL,PIA634,LR,C60-500             			1

503181          DRIVE,PIA634,NO-DRIVE,PAN ONLY          		1

503194          VENT INSTL,P-SIGMA,60"                  			1

CNVR,ROLL,PIA634,LR,C60-500

#### 503127          CNVR,ROLL,PIA634,LR,C60-500

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		  QUANTITY

1       501492          SIDERAIL,REAR,21 ROD,PIA34              			1.

2       501493          SIDERAIL,FRONT,21 ROD,PIA34             			1.

3       023353          PIN,GRAPHITE,1/4ODX1LG                  			2.

4       500932          STRUT,P-SIGMA,STD,TI  *                 			5.

5       500933          STRUT,P-SIGMA,L-R,TI                    				1.

6       501508          SHAFT,LINE,TI,16-9/16                   				1.

7       502995          SHAFT,LINE,TI,18                        				1.

8       501024          GEAR,MITER,13T,PVDF,.929,9P  *          			1.

9       501025          GEAR,MITER,13T,PVDF,1.500,9P *          			5.

10       501026          GEAR,MITER,13T,PVDF,1.662,9P           		          14.

11       501027          GEAR,MITER,13T,BLIND,PVDF,9P *          			1.

27       503130          ROLL,60,2X2,SOLID,LWR,TPN/TI            			7.

ROLL,60,2X2,SOLID,LWR,TPN/T

#### 503130          ROLL,60,2X2,SOLID,LWR,TPN/T

I

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	              QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				1.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              			1.

10       141546          CLIP,RETAINER,PPL                       				1.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

VENT INSTL,P-SIGMA,60"

503194          VENT INSTL,P-SIGMA,60"

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		      QUANTITY

0       503159          PLENUM,PRE634                      				0.00000

0       503188          PLENUM,PLE634                      					0.00000

1       023936          COLLAR,VENT,PVC,MOLDED,4"               			2.

2       011607          DAMPER ASSY,PVC,4"ID                    			2.

3       049817          ADAPTER,VENT,PVC,MOLDED,4"              			2.

4       503195          BAFFLE ASSY,PLENUM,60,P-SIG             			1.

5       002711          SCREW,RD,TI,1/4-20 X 3/4"               				8.

PRE634        Unload & Drive

PRE634M         PRE634,MEC PARTS,P-SIGMA

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	QUANTITY

1       503160          CHAMBER,PRE634                          			1.

3       502770          COVER,SIDERAIL,FRNT,PRE334              		1.

7       502771          COVER,SIDERAIL,REAR,PRE334              		1.

5       503159          PLENUM,PRE634                           			1.

6       131644          COVER INSTL,FRONT,33.882                		1.

7       503158          MOD.INT.PARTS,P-SIGMA,60,TI             		1.

136187          LABELS,SIGMA,TLE\TRE,STD                		1

503131          CNVR,ROLL,PRE634,LR,C60-500             		1

503154          DRIVE,PRE634,140IPM,TI,C60-500          		1

503164          DRAIN BACK,PLE/PRE-PIA-POS,1"           		1

CNVR,ROLL,PRE634,LR,C60-500

#### 503131          CNVR,ROLL,PRE634,LR,C60-500

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        		 QUANTITY

1       501492          SIDERAIL,REAR,21 ROD,PIA34              			1.

2       501493          SIDERAIL,FRONT,21 ROD,PIA34             			1.

3       023353          PIN,GRAPHITE,1/4ODX1LG                  			2.

4       500932          STRUT,P-SIGMA,STD,TI  *                 			5.

5       500933          STRUT,P-SIGMA,L-R,TI                    				1.

6       502998          SHAFT,LINE,TI,17-9/16                   				1.

7       502999          SHAFT,LINE,TI,17-3/16                   				1.

8       501024          GEAR,MITER,13T,PVDF,.929,9P  *          			1.

9       501025          GEAR,MITER,13T,PVDF,1.500,9P *          			5.

10       501026          GEAR,MITER,13T,PVDF,1.662,9P           		          13.

11       501060          GEAR,MITER,13T,PVDF,1.474,9P *          			1.

18       501064          GEAR,SPUR,11T,1/2ID,PVDF     *          			1.

20       501065          PLATE,SKID,DRIVELINE         *         			1.0

21       003615          SCREW,PAN,SST18-8,10-32 X 1/4"         			2.0

23       503130          ROLL,60,2X2,SOLID,LWR,TPN/TI            			7.

ROLL,60,2X2,SOLID,LWR,TPN/T

#### 503130          ROLL,60,2X2,SOLID,LWR,TPN/T

I

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        	              QUANTITY

1       501536          ROLL,D,61-3/4X1.9,TPN                   				1.

3       501015          INSERT,SRAIL,SLTD1-15/16,PVDF*          			1.

4       501019          GEAR,MITER,SPUR,13T,PVDF .920*          			1.

5       149401          SHAFT,PIN,TI,3/8X3-15/16LG *            			1.

6       149399          SHAFT,PIN,TI,3/8X2-7/8LG *              			1.

10       141546          CLIP,RETAINER,PPL                       				1.

11       501016          INSERT,SRAIL,BLIND1-15/16PVDF*          			1.

DRIVE,PRE634,140IPM,TI,C60-500

#### 503154          DRIVE,PRE634,140IPM,TI,C60-500

ITEM_#_ON

DRAWING    PART_NUMBER     DESCRIPTION                        			  QUANTITY

1       502383          MOUNT,MOTOR,DRIVE,PIA*34                				1.

2       067264          PVC,1/2 X  3                        						34.000

2       067264          PVC,1/2 X  3                        						34.000

3       035566          SCREW,FLAT,SST316,1/4-20X1             				4.0

4       502361          PLATE,MTG,TENSION ADJ,P-S,TI            				1.

5       003019          BOLT,HEX,TI,5/16-18X3/4                 				3.

6       003055          WASHER,FLAT,TI,5/16"NOM                 				3.

7       502362          PLATE,TENSION ADJUST,P-SIG,TI           				1.

8       114593          REDUCER,SPEED,60:1                     				1.0

9       105659          MOTOR,DC,1/4 HP,TACH-7V/K,CE *         				1.0

10       003019          BOLT,HEX,TI,5/16-18X3/4                 				4.

11       003055          WASHER,FLAT,TI,5/16"NOM                 				4.

12       013155          WASHER,LOCK,SST316,5/16"NOM            			4.0

13       100128          BOLT,HEX,TI,1/4-20X3                    					2.

15       003056          WASHER,FLAT,TI,1/4 NOM                  				2.

16       026866          WASHER,LOCK,SST316,1/4"NOM             				2.0

17       003032          NUT,HEX,TI,1/4-20                       					2.

18       087396          SPROCKET,BELT,HTD,BLK,5/8ID,24         				1.0

19       170135          KEY,SQ TI,3/16X3/4LG                    					1.

20       080853          BELT,HTD,1040-8M-20                    					1.0

21       501087          GEAR,DRIVE,PVDF,9P,1.662,24T           				1.0

22       501085          SUPPORT,DRIVE,SIDERAIL,P-SIGMA         			2.0

23       501229          WASHER,DOMED,SST303,5/16-NOM            			2.

26       503155          PAN,PRE634,OUTPUT DRIVE                 				1.

27       501230          COVER/BASE PAN SUPPORT,P-SIGMA         			1.0

28       137354          SHEET,PVC,EUROGRY,1/4X48X100IN        			24.0

29       069565          PVC,1/2 X     1/2                     					24.0

30       067246          PVC,1/2 X  6                        						65.750

31       069686          PVC,1/2 X  1                        						65.750

31       069686          PVC,1/2 X  1                        						65.750

General Parts Ordering Instructions

# To order replacement parts refer to the appropriate section of Chapter 9, locate the desired part, its part number, and its name and description.

WHEN ORDERING REPLACEMENT PARTS, SELECT THOSE PART NUMBERS, WHICH PERTAIN TO THE CONVEYOR SIZE AND THE PROCESS STATIONS IN YOUR SYSTEM.

IMPORTANT: WHEN ORDERING REPLACEMENT PARTS, ALWAYS INDICATE SYSTEM SERIAL NUMBER, MODULE SERIAL NUMBER, PART NAME, PART NUMBER, PART DESCRIPTION, AND EQUIPMENT CONVEYOR WIDTH.  CONTACT THE CUSTOMER SERVICE PARTS DEPARTMENT FOR PLACING OF ORDERS.   (SEE FOLLOWING PAGE)

NOTE: ELECTRICAL PART NUMBERS: Part numbers for electrical parts used in this system are shown on the electrical schematic and layout drawings provided with your equipment.  These drawings are placed inside the remote electrical enclosure when your system is ready for shipment from Chemcut.  Refer to these electrical drawings to locate part numbers for the electrical components in your system.

IMPORTANT: THE ELECTRICAL SCHEMATICS AND LAYOUTS SHIPPED WITH YOUR SYSTEM ARE THE ONLY DOCUMENTATION FOR THE PARTS USED IN THE ELECTRICAL CONTROL SYSTEM.  At least one set of these electrical drawings should be kept with the instruction manual to be used as your electrical parts reference.

Chemcut makes every effort to maintain an adequate supply of replacement parts for all of our customer's needs.  For fastest response of your parts order or for technical information, refer to the following page for our 1-800 number.

Parts Ordering Call 1-800-243-6288

# Ordering Parts & Service for your

# Wet Processing Equipment

Is now easier than ever!

# Call 1-800-243-6288

# It's as simple as that!

Chemcut Corp. manufactures and services quality WET PROCESSING EQUIPMENT.

No matter where you are, Quality Original Equipment Parts & Factory Trained Service for your Chemcut Equipment is but a phone call away!

Your call will be automatically routed to ensure that you will receive fast, courteous service when you want it.

And, we offer full service and maintenance on every make of wet processing equipment too! Give it a try; we're here to keep you in production.

# CHEMCUT CORP. • 500 Science Park • State College, PA 16803

Tel (814) 272-2800 • FAX (814) 272-5208

Recommended Spare Parts

SYSTEM C60-500

It is recommended that the parts listed in this section be carried as spares by users of system C60-500. The information provided consists of recommended stocking quantity for each part, CHEMCUT part number for each part, and name and description of each part.

Part Number 	                       Description	         Qty.	     Unit$        Ext$

007526         VALVE,SOL,PAINT,BRS,1/2NPT,N/C      1   222.00   222.00

011905         VALVE,SOL,PAINT,BRS,3/4NPT,N/C      1   259.00   259.00

012758         HYDROMETER,SPECIFIC-GRAVITY         1    25.50    25.50

018836         SEAL,O-RING,2-330,EPDM,BLACK        1     5.40     5.40

019748         SEAL,O-RING,2-121,VITON,BROWN      10      .65     6.50

023082         LAMP,INCANDESCENT                   1     2.95     2.95

023651         WASHER,THRUST,TFE,.030              7     1.65    11.55

023759         FUSE,CART,FNM-5 A,250 VOLT          1     3.90     3.90

027320         SCREW,CAP,PVC,5/16-18X1/2"LG        1     3.85     3.85

029913         HOUSING,MAGNET,PVC,3/8              1    21.60    21.60

031926         SEAL,O-RING,2-232,EPDM,BLACK        1     2.15     2.15

034665         SWITCH ASSY,REED,PVC,12'            1    49.00    49.00

035031         SEAL,V-RING,V-12S,NITRILE           1     3.70     3.70

040107         SWITCH,LIQUID LEVEL,PPL,COMPAC      1    40.50    40.50

043105         SOCKET,RELAY BLADE TYPE             1    19.10    19.10

043734         INSERT,SIDERAIL,RPE,3/8DIA          8     5.10    40.80

044191         RELAY,4PDT,120VAC-C                 1    15.00    15.00

044192         RELAY,4PDT, 24 VDC-C                1    13.60    13.60

044426         INSERT,SIDERAIL,RPE,3/8,DEWEBB      7     5.95    41.65

044648         GEAR,SPUR,RPE,15T,EXT               4     5.75    23.00

044652         INSERT,SIDERAIL,RPE,DE-WEBBED       1     7.45     7.45

046372         SWITCH ASSY,EMERGENCY STOP          1    57.50    57.50

046914         FUSE,CART,FNM-2 A,250 VOLT          1     8.65     8.65

073711         NOZZLE,SCRD,PVDF,1.5GPM,70DEG      10     8.05    80.50

075246         SEAL,O-RING,2-209,EPDM,BLACK       20      .60    12.00

080192         SWITCH,FLUID LEVEL,CS/SIGMA         1    71.00    71.00

081141         SWITCH ASSY,ILLUMINATED,STOP        1    55.00    55.00

081142         SWITCH ASSY,ILLUMINATED,START       1    55.00    55.00

084577         CARAPACE SUBASS,LARGE,RPP,NPT       1   268.00   268.00

088395         CAP,PIPE,PVC,MACHINED               3    15.95    47.85

103650         GAUGE,PRESS,0-60/4BAR,W/ISOL        1   102.00   102.00

109975         PIN ASSY,INT GEAR,RPP               1    20.80    20.80

114264         CONTROLLER,DC MOTOR SPEED           1   145.00   145.00

115941         FUSE,CART,ABC-10 A,250 VAC          1     2.40     2.40

116673         INSERT,SIDERAIL,RPE,BLIND          21     5.30   111.30

117858         SENSOR,PHOTOELECTRIC,10-30 VDC      1   153.00   153.00

118289         FUSE,CART,CLASS J,10A,600 VOLT      1    11.90    11.90

118290         FUSE,CART,CLASS J,15A,600 VOLT      1    11.90    11.90

118291         FUSE,CART,CLASS J,20A,600 VOLT      1     9.55     9.55

119055         FUSE,CART,ABC-2 A,250 VOLT          1     3.20     3.20

123440         CONTACTOR,IEC,30A,10HP,AB           1    77.50    77.50

125942         METER,LOOP POWERED,3-1/2 LCD        1   173.00   173.00

128115         TRANSMITTER,ISOLATOR,UNIVERSAL      1   492.00   492.00

146043         RELAY,OVERLOAD,MCS 1-2.9A,AB        1    53.50    53.50

146045         RELAY,OVERLOAD,MCS 3.7-12A,AB       1    53.00    53.00

148916         KEY,SQ TI,3/16X1-1/2LG              1     7.60     7.60

156728         GEAR,SPUR,RPP,15T,3/8,MOD           1     7.25     7.25

164282         TIMER,OMRON,24-230V AC/DC           1   108.00   108.00

164422         PROBE,T-S/OT,132 F,14 IN,POLY       1   165.00   165.00

170033         CAM,OSC,5762,TI                     1   167.00   167.00

170034         COUPLING,DRIVE,OSC,TI               1   171.00   171.00

170069         GEAR,DRIVE,PVC,8P,2.000,24T         1    15.70    15.70

170076         BLOCK,BEARING,RPE/TI                2    24.55    49.10

170131         SPRAYTUBE ASSY,PEM,XL               1    96.50    96.50

170173         LINK,OSCILLATION,RPP,XL             7    19.60   137.20

170787         MOTOR,GEAR,DC,1/8HP,30RPM,TACH      1   626.00   626.00

402227         SPROCKET,BELT,HTD,BLK,3/4ID,24      1    98.00    98.00

600521         INSERT,SIDERAIL,1.437,DEWEBB        1    13.95    13.95

600523         INSERT,SIDERAIL,RPE,3/8,BLIND       1    10.50    10.50

600590         GEAR,MITER/SPUR,RPE,"D",1.419       2     2.60     5.20

600592         GEAR,MITER,LINE,RPP,13T,XL   *      6     3.95    23.70

Total cost:                  4595.45

General Parts Ordering Instructions

# To order replacement parts refer to the appropriate section of Chapter 9, locate the desired part, its part number, and its name and description.

WHEN ORDERING REPLACEMENT PARTS, SELECT THOSE PART NUMBERS, WHICH PERTAIN TO THE CONVEYOR SIZE AND THE PROCESS STATIONS IN YOUR SYSTEM.

IMPORTANT: WHEN ORDERING REPLACEMENT PARTS, ALWAYS INDICATE SYSTEM SERIAL NUMBER, MODULE SERIAL NUMBER, PART NAME, PART NUMBER, PART DESCRIPTION, AND EQUIPMENT CONVEYOR WIDTH.  CONTACT THE CUSTOMER SERVICE PARTS DEPARTMENT FOR PLACING OF ORDERS.   (SEE FOLLOWING PAGE)

NOTE: ELECTRICAL PART NUMBERS: Part numbers for electrical parts used in this system are shown on the electrical schematic and layout drawings provided with your equipment.  These drawings are placed inside the remote electrical enclosure when your system is ready for shipment from Chemcut.  Refer to these electrical drawings to locate part numbers for the electrical components in your system.

IMPORTANT: THE ELECTRICAL SCHEMATICS AND LAYOUTS SHIPPED WITH YOUR SYSTEM ARE THE ONLY DOCUMENTATION FOR THE PARTS USED IN THE ELECTRICAL CONTROL SYSTEM.  At least one set of these electrical drawings should be kept with the instruction manual to be used as your electrical parts reference.

Chemcut makes every effort to maintain an adequate supply of replacement parts for all of our customer's needs.  For fastest response of your parts order or for technical information, refer to the following page for our 1-800 number.

Parts Ordering Call 1-800-243-6288

# Ordering Parts & Service for your

# Wet Processing Equipment

Is now easier than ever!

# Call 1-800-243-6288

# It's as simple as that!

Chemcut Corp. manufactures and services quality WET PROCESSING EQUIPMENT.

No matter where you are, Quality Original Equipment Parts & Factory Trained Service for your Chemcut Equipment is but a phone call away!

Your call will be automatically routed to ensure that you will receive fast, courteous service when you want it.

And, we offer full service and maintenance on every make of wet processing equipment too! Give it a try; we're here to keep you in production.

# CHEMCUT CORP. • 500 Science Park • State College, PA 16803

Tel (814) 272-2800 • FAX (814) 272-5208

Recommended Spare Parts

SYSTEM C60-500

It is recommended that the parts listed in this section be carried as spares by users of system C60-500. The information provided consists of recommended stocking quantity for each part, CHEMCUT part number for each part, and name and description of each part.

Part Number 	                       Description	         Qty.	     Unit$        Ext$

007526         VALVE,SOL,PAINT,BRS,1/2NPT,N/C      1   222.00   222.00

011905         VALVE,SOL,PAINT,BRS,3/4NPT,N/C      1   259.00   259.00

012758         HYDROMETER,SPECIFIC-GRAVITY         1    25.50    25.50

018836         SEAL,O-RING,2-330,EPDM,BLACK        1     5.40     5.40

019748         SEAL,O-RING,2-121,VITON,BROWN      10      .65     6.50

023082         LAMP,INCANDESCENT                   1     2.95     2.95

023651         WASHER,THRUST,TFE,.030              7     1.65    11.55

023759         FUSE,CART,FNM-5 A,250 VOLT          1     3.90     3.90

027320         SCREW,CAP,PVC,5/16-18X1/2"LG        1     3.85     3.85

029913         HOUSING,MAGNET,PVC,3/8              1    21.60    21.60

031926         SEAL,O-RING,2-232,EPDM,BLACK        1     2.15     2.15

034665         SWITCH ASSY,REED,PVC,12'            1    49.00    49.00

035031         SEAL,V-RING,V-12S,NITRILE           1     3.70     3.70

040107         SWITCH,LIQUID LEVEL,PPL,COMPAC      1    40.50    40.50

043105         SOCKET,RELAY BLADE TYPE             1    19.10    19.10

043734         INSERT,SIDERAIL,RPE,3/8DIA          8     5.10    40.80

044191         RELAY,4PDT,120VAC-C                 1    15.00    15.00

044192         RELAY,4PDT, 24 VDC-C                1    13.60    13.60

044426         INSERT,SIDERAIL,RPE,3/8,DEWEBB      7     5.95    41.65

044648         GEAR,SPUR,RPE,15T,EXT               4     5.75    23.00

044652         INSERT,SIDERAIL,RPE,DE-WEBBED       1     7.45     7.45

046372         SWITCH ASSY,EMERGENCY STOP          1    57.50    57.50

046914         FUSE,CART,FNM-2 A,250 VOLT          1     8.65     8.65

073711         NOZZLE,SCRD,PVDF,1.5GPM,70DEG      10     8.05    80.50

075246         SEAL,O-RING,2-209,EPDM,BLACK       20      .60    12.00

080192         SWITCH,FLUID LEVEL,CS/SIGMA         1    71.00    71.00

081141         SWITCH ASSY,ILLUMINATED,STOP        1    55.00    55.00

081142         SWITCH ASSY,ILLUMINATED,START       1    55.00    55.00

084577         CARAPACE SUBASS,LARGE,RPP,NPT       1   268.00   268.00

088395         CAP,PIPE,PVC,MACHINED               3    15.95    47.85

103650         GAUGE,PRESS,0-60/4BAR,W/ISOL        1   102.00   102.00

109975         PIN ASSY,INT GEAR,RPP               1    20.80    20.80

114264         CONTROLLER,DC MOTOR SPEED           1   145.00   145.00

115941         FUSE,CART,ABC-10 A,250 VAC          1     2.40     2.40

116673         INSERT,SIDERAIL,RPE,BLIND          21     5.30   111.30

117858         SENSOR,PHOTOELECTRIC,10-30 VDC      1   153.00   153.00

118289         FUSE,CART,CLASS J,10A,600 VOLT      1    11.90    11.90

118290         FUSE,CART,CLASS J,15A,600 VOLT      1    11.90    11.90

118291         FUSE,CART,CLASS J,20A,600 VOLT      1     9.55     9.55

119055         FUSE,CART,ABC-2 A,250 VOLT          1     3.20     3.20

123440         CONTACTOR,IEC,30A,10HP,AB           1    77.50    77.50

125942         METER,LOOP POWERED,3-1/2 LCD        1   173.00   173.00

128115         TRANSMITTER,ISOLATOR,UNIVERSAL      1   492.00   492.00

146043         RELAY,OVERLOAD,MCS 1-2.9A,AB        1    53.50    53.50

146045         RELAY,OVERLOAD,MCS 3.7-12A,AB       1    53.00    53.00

148916         KEY,SQ TI,3/16X1-1/2LG              1     7.60     7.60

156728         GEAR,SPUR,RPP,15T,3/8,MOD           1     7.25     7.25

164282         TIMER,OMRON,24-230V AC/DC           1   108.00   108.00

164422         PROBE,T-S/OT,132 F,14 IN,POLY       1   165.00   165.00

170033         CAM,OSC,5762,TI                     1   167.00   167.00

170034         COUPLING,DRIVE,OSC,TI               1   171.00   171.00

170069         GEAR,DRIVE,PVC,8P,2.000,24T         1    15.70    15.70

170076         BLOCK,BEARING,RPE/TI                2    24.55    49.10

170131         SPRAYTUBE ASSY,PEM,XL               1    96.50    96.50

170173         LINK,OSCILLATION,RPP,XL             7    19.60   137.20

170787         MOTOR,GEAR,DC,1/8HP,30RPM,TACH      1   626.00   626.00

402227         SPROCKET,BELT,HTD,BLK,3/4ID,24      1    98.00    98.00

600521         INSERT,SIDERAIL,1.437,DEWEBB        1    13.95    13.95

600523         INSERT,SIDERAIL,RPE,3/8,BLIND       1    10.50    10.50

600590         GEAR,MITER/SPUR,RPE,"D",1.419       2     2.60     5.20

600592         GEAR,MITER,LINE,RPP,13T,XL   *      6     3.95    23.70

Total cost:                  4595.45

Warranty

# Chemcut Corp. TERMS AND CONDITIONS OF SALE - ELECTRONICS EQUIPMENT GROUP

These are the terms and conditions of sale governing the transaction on front hereof.  The following terms and conditions of sale supersede any conflicting terms included in Buyer’s purchase order.

1.   Drawings, Specifications, Confidentiality and Trademarks

Any drawings, specifications or descriptions accompanying the quotation are estimates and approximations and do not represent the terms of the agreement between Buyer and Chemcut.  Any drawings, documentation or information delivered with a quotation are the property of Chemcut and shall be held in confidence by Buyer.  Buyer agrees to protect the trademarks of Chemcut and not to remove or deface such trademarks affixed to any goods or equipment sold by Chemcut.

2.   Delivery

All sales by Chemcut are F.O.B. point of shipment.  By accepting delivery F.O.B. at point of shipment, Buyer acknowledges that it shall provide, at its cost, all facilities for shipment, insurance, unloading and installation of equipment sold by Chemcut.  All reasonable means will be used to make shipment at the time specified, but we assume no liability for loss or damage arising from delays due to fires, strikes, or any causes beyond our control.  Quoted deliveries from stock are subject to prior sale.

3.   Payment

Terms of payment shall be net 30 days from date of invoice.

Should buyer for any reason fail to make timely payments, without affecting any other right which Chemcut may have, Chemcut shall be entitled to interest accruing at the rate which is the lower of 1% per month or the highest aggregate rate of interest permitted by law.  Chemcut shall also have the right to offset the outstanding balance against any payments due by Chemcut.

4.   Security Interest in Goods Sold

Upon placing an order for goods or equipment, Buyer grants to Chemcut a security interest in the goods or equipment purchased as security for the satisfaction of Buyer’s account balance.  Buyer shall deliver to Chemcut at Chemcut’s request, such executed financing statements, continuation statements and other documents as are required to perfect, preserve and continue Chemcut’s security interest in such goods or equipment.  Upon default in the payment terms agreed upon by the parties, Chemcut may declare all obligations secured hereby immediately due and payable and shall have the remedies of a secured party under the Uniform Commercial Code.

5.   Equipment and Parts Warranty

There are no warranties, expressed or implied by either distributor or the manufacturer on new equipment except the manufacturer’s warranty against defects in material and workmanship set forth below:

Chemcut warrants new equipment manufactured by Chemcut and delivered to the original retail purchaser F.O.B. Chemcut’s factory, to be free from defects in material and workmanship under normal use and service, for a period of one year from delivery of the equipment or for the first 2000 hours of operation whichever occurs earlier.  Chemcut warrants parts for thirty (30) days from the date of purchase.  These warranties are subject to the following limitations to which the Buyer expressly agrees:

Chemcut’s obligation under this warranty is limited solely to repairing or replacing, at our option and without  charge, FOB Chemcut’s factory  in  State College, PA with  transportation prepaid by the Buyer  within the warranty period and which, upon examination by Chemcut shall be found to be reasonable satisfaction to have been thus defective.  THIS REMEDY IS EXPRESSLY SUBSTITUTED FOR ANY AND ALL OTHER  REMEDIES POSSIBLE UNDER THE UNIFORM COMMERCIAL CODE, STATE,  COMMON OR STATUTORY LAW OR OTHERWISE.

B.   The provisions of this warranty SHALL NOT APPLY:

To any equipment which has been subject to misuse, negligence or accident or which has been repaired or altered outside      Chemcut’s factory in any way so as to, in Chemcut’s sole but reasonable judgement, affect its performance and reliability.

To any equipment or parts that are used with chemistry or temperature that is not compatible with the materials of   construction.  Compatibility of chemistry and temperature must be approved in writing by Chemcut..

To any equipment and parts that are operated with the safety interlock system disconnected or bypassed.

To any equipment or parts manufactured pursuant to design or specifications supplied by the Buyer which have not been formally accepted by Chemcut.

To any equipment, parts or components which, under normal usage, would not or are not expected to last the warranty period, i.e. “wear” items (i.e. gears, sprockets, bearings, gear couplings, nozzles, oscillation assemblies, manifold packing, and manifold adapters).

To any consumable parts or components with limited use potential, (i.e. water filters, air filters, scrubbing/deburring brushes, lamps, filters, thin material parts, drive belts, probes and brushes).

To any equipment and parts which have not been subject to proper maintenance as set forth in the maintenance schedule provided.  Maintenance records should be kept for verification.

C.	Chemcut shall not be liable for any damages, whether direct or indirect, economic, commercial, incidental, or consequential, and whether arising from Chemcut’s negligence, breach of contract, product liability, warranty or any other reason.

- The 2000 hours of operation shall be determined solely by a timing device, which is built in the equipment.  If, upon examination by Chemcut, it is determined that this timing device or its electrical connections have been tampered with to delay the expiration of this warranty, this warranty shall be immediately null and void.

- THIS WARRANTY IS EXPRESSLY IN LIEU OF ANY OTHER WARRANTIES, EXPRESSED OR IMPLIED INCLUDING ANY IMPLIED OR EXPRESSED WARRANTY OF MERCHANTABILITY, SUITABILITY OR FITNESS FOR A PARTICULAR PURPOSE and Chemcut neither assumes nor authorizes another to assume any liability in connection with such equipment, except as provided above.

F.	This warranty does not apply to equipment not manufactured by Chemcut.  For equipment furnished, but not manufactured by Chemcut,  Chemcut assigns to the Buyer any  warranty and/or  claim it may have against the manufacturer or supplier of  the equipment.

6.     Taxes and Permits

Buyer shall be responsible for all taxes on the sale, installation and use of goods sold by Chemcut.  Buyer shall be responsible for obtaining  all necessary permits and approvals.

7.	Termination

Accepted purchase orders can be terminated only with the consent of Chemcut  and in the event of such termination, Chemcut shall be entitled to recover its costs incurred as a result of its performance, including administrative, labor and materials, and a reasonable profit.  Chemcut shall have the option to terminate a purchase order in the event of Buyer’s dissolution, insolvency, or business failure, the appointment of a custodian, liquidator, receiver, or trustee of or for any part of Buyer’s property, an assignment for the benefit of creditors by, or the commencement of any proceedings under any bankruptcy or insolvency law by or against the Buyer.

8.	Change Orders; Modifications

Any change orders requested by Buyer shall be in writing and shall be treated as additional purchase orders, subject to acceptance by Chemcut.

9.	Field Service

The schedule and cost of any field service to be provided by Chemcut shall be stated in the quotation.  Chemcut will provide additional field service only upon its acceptance of an order for service and at its rates prevailing at the time.

10.	Installation Checklist

Installation checkout is provided with the purchase of new Chemcut systems.  This is performed by a qualified Chemcut representative after the system is completely installed to insure proper start up of the system.  Included with the checkout is a training session of up to (4) hours  in operation, care and maintenance of the system, both electrical and mechanical.  NOTE:  Additional training is available at a cost  dependent upon the complexity of the system.  For available training courses and cost information, please contact the Customer Service Department at Chemcut, State College at (814) 238-0514 or 1-800-243-6288.

11.	Manuals and Electrical Schematics

One standard Chemcut equipment manual and one (1) set of electrical schematics are provided with

every new Chemcut system.

12.	Clerical Errors

Chemcut reserves the right to correct clerical errors.

13.	Force Majeure

Chemcut shall not be liable for any damage or penalty for delays in delivery, installation, complying with the warranty provisions hereunder due to acts of God, acts or omissions of the customer, acts of Civil or Military authorities, government regulations or priorities, fires, floods, epidemics, quarantine restrictions, war, riots, strikes, differences with workmen, accidents to machinery, inability to obtain the necessary materials of manufacturing facilities, delays in transportation, failure of or delay in furnishing correct or complete information by customer with respect to details of work to be performed hereunder, impossibility of performance or any other cause or causes beyond the control of Chemcut, whether or not similar to the foregoing.  In the event of any delay caused bas aforesaid, the delivery, installation, completion and/or warranty performance data shall be extended for a period equal to any such delay, and this contract shall not be void or voidable as a result of any such delay.  CHEMCUT SHALL UNDER NO CIRCUMSTANCES BE LIABLE FOR SPECIAL OR CONSEQUENTIAL DAMAGES FOR DELAYS OR FAILURE TO GIVE NOTICE OF DELAY.

14.	Modifications; Governing Law

The Agreement represented by an accepted purchase order may be modified only by a written agreement between Chemcut and Buyer.  Such agreement will be governed by and construed in accordance with Pennsylvania Law.  If any provision of the agreement between Chemcut  and Buyer shall be held illegal, invalid or unenforceable in any jurisdiction, it shall be ineffective in such jurisdiction only to the extent of  such illegality, invalidity or unenforceability, and such illegality invalidity or unenforceability shall not invalidate the remaining provisions of such agreement.

15.   Claims

- Buyer shall have no right to reject nonconforming goods or to rescind, but BUYER’S EXCLUSIVE REMEDY SHALL BE A CLAIM FOR MONETARY ADJUSTMENT,, or al Seller’s option, shipment of conforming goods and return of disputed goods to Chemcut.  Buyer shall afford Chemcut prompt and reasonable opportunity to inspect all material as to which any claim is made.  If  Chemcut and Buyer are unable to reach settlement of any claim relating to material covered hereby.  Buyer must institute legal action against Chemcut within one (1) year after such claim arises and thereafter all such claims shall be barred notwithstanding any  statutory period of limitation to the contrary.

- CHEMCUT’S LIABLITY ON ANY CLAIM, WHETHER BASED IN NEGLIGENCE, TORT,

CONTRACT OR OTHERWISE,  WITH RESPECT TO MATERIAL DELIVERED HEREUNDER

OR TO NONDELIVERY OF  MATERIAL, SHALL NOT EXCEED THE PURCHASE PRICE OF

THAT PORTION OF THE MATERIAL IN RESPECT OF WHICH CLAIM IS MADE.  IN NO

EVENT SHALL SELLER BE LIABLE FOR INCIDENTAL OR CONSEQUENTIAL DAMAGES.

Buyer assumes all risk of  loss and liability from use or resale of any material delivered.

16.   Equipment and Parts Return

All returns of equipment and/or spare parts will be subject to a 25% restocking fee.

QD  1072			REV #1			REV. DATE 5-1-2002

Chemcut Contacts

# Contents..………………..……………………………………………………………….Page

11.1	General	233

11.2	State College contact information	233

11.3	Eastern Region, U.S. contact information	234

11.4	Central Region, U.S. contact information	235

11.5	Western Region, U.S. contact information	236

11.6	Europe Region contact information	237

11.7	Asia Region contact information	238

General

Chemcut Corp. main equipment facility is located in State College PA.   This facility contains the sales, purchasing, engineering, manufacturing, information systems, customer service, (parts), and technical support functions for Chemcut Corp.

equipment business.

In addition to the State College facility, several regional sales and service offices exist throughout the world.  The United States is divided into 3 regions (East, Central, and West).   Europe and Asia are also considered regions.  Each of these 5 regions, has sales and/or service organizations to directly support customers within the geographic region.

The information contained in this section of the technical documentation, is intended to identify the proper contact point within the Chemcut Corp. organization.

State College contact information

Address:

Chemcut Corp.

500 Science Park Road

State College  PA

16803

Phone:

814-272-2800  (main switchboard, follow the automated menu)

    - (toll free within the U.S.)

Email:

customer.support@chemcut.net  (support)

marketing@chemcut.net (request sales literature)

tech.support@chemcut.net (technical assistance)

parts@chemcut.net (parts)

Eastern Region, U.S. contact information

Phone:

814-272-2800  (main switchboard, follow the automated menu)

800-243-6288 (toll free within the U.S., follow the automated menu)

Email:

sales.east@chemcut.net(sales)

service.east@chemcut.net (service)

customer.support@chemcut.net  (support)

tech.support@chemcut.net (technical assistance)

parts@chemcut.net (parts)

Central Region, U.S. contact information

Phone:

814-272-2800  (main switchboard, follow the automated menu)

800-243-6288 (toll free within the U.S., follow the automated menu)

Email:

sales.central@chemcut.net (sales)

service.central@chemcut.net (service)

customer.support@chemcut.net  (support)

tech.support@chemcut.net (technical assistance)

parts@chemcut.net (parts)

Western Region, U.S. contact information

Phone:

814-272-2800  (main switchboard, follow the automated menu)

800-243-6288 (toll free within the U.S., follow the automated menu)

Email:

sales.west@chemcut.net (sales)

service.west@chemcut.net (service)

customer.support@chemcut.net  (support)

tech.support@chemcut.net (technical assistance)

parts@chemcut.net (parts)

Europe Region contact information

Phone:

001-814-272-2800  (main switchboard, follow the automated menu)

Email:

sales.europe@chemcut.net (sales)

service.europe@chemcut.net (service)

customer.support@chemcut.net  (support)

tech.support@chemcut.net (technical assistance)

parts@chemcut.net (parts)

Asia Region contact information

Phone:

001-814-272-2800  (main switchboard, follow the automated menu)

Email:

sales.asia@chemcut.net (sales)

service.asia@chemcut.net (service)

customer.support@chemcut.net  (support)

tech.support@chemcut.net (technical assistance)

parts@chemcut.net (parts)