## Technical Bulletin

Safe Fleet

6800 E 163rd St

Belton, MO  64012-5463

816-318-8000

800-827-3692

Fax  816-318-8100

#### DuroLumen V4 LED Interior Lighting

FM-7.5-354

Rev B

07-25-14

Our Technical Bulletins are provided to inform you of design options, design improvements,

standard sizes and changes, different models, applications, installation, operation, and maintenance.

R•O•M is proud to announce the DuroLumen V4 LED Interior Light as a replacement for the DuroLumen V3.  The DuroLumen V4 is available in both recessed and surface mounts, with two different recessed mounting options.

| Cross Reference Between DuroLumen V3 and DuroLumen V4 models | NEW DuroLumen V4 |  |  | DuroLumen V3 Obsolete Part # |
| --- | --- | --- | --- | --- |
|  | Surface Mount Part # | Emergency Recessed Mount Part # | Truck & Trailer Recessed Mount Part # |  |
| DuroLumen - White LED Interior Light 6 LED Array / 1400 Lumens / Draw 1.25A @ 12.6V DC | R03694 | R03851 | R03848 | R02825 R03219 |
| DuroLumen - White / White LED Interior Light can be switched between 6 & 12 LEDs Arrays after install * 6 LED Array / 1400 Lumens / Draw 1.25A @ 12.6V DC & 12 LED Array / 2800 Lumens / Draw 2.50A @ 12.6V DC | R03696 | R03853 | R03850 | R03063 R03215 |
| DuroLumen Red / White LED Interior Light can be switched between Red or White LEDs after install * 6 White LED Array / 1400 Lumens / Draw 1.25A @ 12.6V DC & 6 Red LED Array / 500 Lumens / Draw 1.25A @ 12.6V DC | R03695 | R03852 | R03849 | R02986 R03218 |

* switch not provided

| Features: | Operating Voltage |  | 9.0 V DC to 16 V DC |
| --- | --- | --- | --- |
|  |  |  | Lumens remain constant with voltage variation through this range |
|  | Weight |  | Lightweight - Approx. 1.1 lb (surface mount) / 1.75 lbs. (recessed mount) |
|  | Lamp Life |  | Up to 50,000 hours and maintenance free |
|  | Wire Gage |  | 18 AWG |
|  | Operating Temperature |  | -25˚F to 150˚F |
|  |  |  | Robust aluminum housing |
|  |  |  | LED board directly mounted to housing for natural heat sink |
|  | Dimensions / Mounting options |  | Emergency Recessed Mount - 5.8" W x 17.9" L x 0.8" D with mounting holes |
|  |  |  | Truck & Trailer Recessed Mount - 5.8" W x 17.9" L x 0.8" D with mounting slots |
|  |  |  | Surface Mount - 3.2" wide x 15.5" long x 0.8" deep |
|  | Retrofittable |  | Recessed Mount easily retrofits to existing trailer fluorescent light fixture cutouts |
|  |  |  | - Cutout / Opening dimensions for recessed mount : 3.5" wide x 16" long x 1" deep |
|  |  |  | Must be installed in a current limiting (fused) circuit |
|  | Durability |  | LEDs are more robust than fluorescents against vibration and the tough |
|  |  |  | Application of this environment. Solid states means no moving parts. |
|  | Better Lighting |  | Reflective housing design |
|  |  |  | Wide Viewing Angle for efficient distribution |
|  |  |  | Solid State (no moving parts & no flickering), tested to military standard (MIL 810-A). |
|  | Color |  | CCT (Correlated Color Temperature) Tolerance to insure bright white (or red) |
|  |  |  | color consistency for true surface color reflection (White LEDs) |
|  | Moisture |  | IPX7 rating: Withstands a 30-minute water submersion test while operating |
|  | Warranty |  | 7-Year warranty |

For additional questions, please contact your R•O•M Customer Development Representative at (800) 827-3692.